VIETNAM NATIONAL UNIVERSITY

HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY

FACULTY OF MECHANICAL ENGINEERING

-----❖-----

PROJECT

TRANSMISSION SYSTEM

Instructor:			Prof. Nguyễn Hữu Lộc

Student’s name: 		Nguyễn Quang Huy 	ID: 2153385

Huỳnh Lê Hùng 		ID: 2152605

PROBLEM 10A: DESIGN OF TRANSMISSION SYSTEMS FOR DRUM DRYING

Hệ thống truyền động bao gồm:

1- Động cơ điện;  2- Nối trục đàn hồi;  3- Hộp giảm tốc bánh răng nghiêng 1 cấp; 4- Bộ truyền xích (u = 1); 5- Bộ truyền bánh răng trụ hở;   6, 7 – Con lăn; 8- Thùng sấy; 9- Nạp liệu. Tỉ số truyền cặp bánh răng thùng trôn u = 10.

Option 6: Design parameters

| Power P (kW) | 2.2 |
| --- | --- |
| Rotational speed (rpm) | 20 |
| Working hour | 8000h |
| Working shift | 1 |

## CHAPTER I: MOTOR SELECTION AND TRANSMISSION RATIO DISTRIBUTION

  - General efficiency of transmission system

According to table 2.3, we choose the efficiencies  of machines elements as follows:

- The efficiency of chain:

ηchain = 0.90

- The efficiency of lubricated gear:

ηgear = 0.97

- The efficiency of open gear:

ηopen gear = 0.94

- The efficiency of rolling-contact bearings:

ηrolling = 0.99

- The efficiency of coupling:

ηcoupling = 0.98

The general efficiency of transmission system:

ηgeneral = ηchain × ηrolling3 × ηgear × ηopen gear  × ηcoupling

ηgeneral = 0.90 × 0.993 × 0.97 × 0.94 × 0.98 = 0.78

  - Required power of motor

- Working power of system:

P = 2.2 kW

- Rotational speed of drum drying:

n = 20 rpm

- Required power of motor:

Pct=Pηgeneral=2.20.78=2.82 kW

  - Motor selection and transmission ratio

According to the calculated required power, we choose the standard motor power as 3.0 kW. With that standard motor power, we have the following options to choose our motor as well as its rotational speed and transmission ratio.

|  | Rotational speed (rpm) | General transmission usys | Chain uch | Spur gear u12 | Open gear u34 |
| --- | --- | --- | --- | --- | --- |
| Motor 1 | 2870 | 143.5 | 1.79 | 8 | 10 |
| Motor 2 | 1420 | 71 | 1.13 | 6.3 | 10 |
| Motor 3 | 970 | 48.5 | 0.97 | 5 | 10 |
| Motor 4 | 715 | 35.75 | 0.89 | 4 | 10 |

Table 1.1: Motor selection and transmission system

Based on table Motor SGA (Company CMG, Australia). We chose Motor 2 with a rotational speed of 1420 rpm.

  - Parameters of transmission system

Power P on shaft

Pmotor =  2.82 kW

PIII=Pηopen gear×ηrolling=2.20.94×0.99=2.364 kW

PII=PIIIηchain×ηrolling=2.3640.9×0.99=2.653 kW

PI=PIIηgear×ηrolling=2.6530.97×0.99=2.763kW

Pmotor=PIηcoupling=2.7630.98=2.82 kW (satisfied)

Rotational speed on shafts:

nmotor=1420 rpm

nI=nmotorucoupling=14201=1420 rpm

nII=nmotorugear=14206.3=225.4 rpm

nIII=nIuchain=225.41.13=199.47 rpm

ndrum=nIIudrum=199.4710=19.95 rpm

Torque on shafts:

Tmotor=9550Pmotornmotor=95502.821420=18.965Nm

TI=9550PInI=95502.7631420=18.582 Nm

TII=9550PIInII=95502.653225.4=112.405 Nm

TIII=9550PIIInIII=95502.364199.47=113.181 Nm

Tdrum =9550Pdrumndrum=95502.219.95=1053.133 Nm

  - End of chapter

We have the following table of parameters of the transmission system:

Table 1.2: Parameters of transmission system

| Parameters | Shaft |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Motor | I |  | II |  | III |  | Working |
| Power P (kW) | 2.82 | 2.763 |  | 2.653 |  | 2.364 |  | 2.2 |
| Ration u | 1 |  | 6.3 |  | 1.13 |  | 10 |  |
| Rotational speed (rpm) | 1420 | 1420 |  | 225.4 |  | 199.47 |  | 19.95 |
| Torque T (Nm) | 18.965 | 18.582 |  | 112.405 |  | 113.181 |  | 1053.133 |

Where:

- Motor Shaft: connecting the motor to the coupling

- Shaft I: connecting the coupling to the single stage helical gear reducer

- Shaft II: connecting the single stage helical gear reducer to the Chain.

- Shaft III: connecting the Chain to the open gear drive.

- Working Shaft: connecting the open gear drive to the drying drum.

## CHAPTER II. DESIGN AND SELECT FOR CHAIN

2.1	Parameters calculated from 1:

Input parameters:  = 3.966 kW, = 120 rpm, u = 2.54

2.2 Preliminary selection of the number of teeth

Number of teeth for driving sprocket:

Number of teeth for driven sprocket:

2.3 Condition factor K calculation

Where:

1 - The using the electric power and load out of the effect up the drives: calm

1 - Factor taking into account the influence of center distance a = (30÷50)pc

1 - Factor taking into account the effect of drive chain arrangement

1 - Adjustable sprocket shaft

1.5 - Factor taking into account the effect of lubrication, in the case of drip lubrication

1.12 - Duty factor, when working 1 shift

From here infer:

2.4 Power calculation

We have:

Where:

Substitute the factors and values into the  formula we get:

2.5 Select chain pitch

According to table 5.4 from file Chapter 5 Chain Drive, respectively  and rotational speed  we select  with

2.6 Checking chain pitch calculation

Where:  is chosen from table 5.3 in file Chapter 5 Chain Drive.

is satisfied

2.7 Center distance a

Choose

2.8 The number of links

The number of links is determined by the formula:

Choose X = 124

2.9 The chain length

2.10	Determine the force on each shaft:

Initial belt tension:

[σo] ≤ 1.5 MPa for V-Belt

F0=Aσ0=zA1σ0=2×138×1.5=414 N

Tension per belt:

F0z=414 2=207 N

Useful force:

Ft=1000P1v1=1000×4.178.04=518.66 N

Useful force per belt:

Ftz=518.66 2=259.33 N

Force acting on shaft:

Fr≈2F0sinsin α12 =2×414×sin161.12=816.76 N

2.11 	Stress in belt drive and service life

Maximum belt stress in belt drive:

σmax=σ1+σv+σu1=σ0+0.5σt+σv+σF1

→σmax=F0A+0.5FtA+ρv2×10-6+2y0dE

→σmax=207 138+0.5259.33138+1200×8.042×10-6+2×4160×100

=7.517 MPa

Service life:

Lh = σrσmaxm×1072 ×3600i=97.5178×1072 ×3600 ×4.02=1458.9 hours

2.12 	Summary

| V-Belt type B |  | Unit |
| --- | --- | --- |
| Small pulley diameter d1 | 160 | mm |
| Big pulley diameter d2 | 355 | mm |
| Center distance a | 587.43 | mm |
| Belt length L | 2000 | mm |
| Contact angle α1 | 161.1 | degree |
| Number of the belts z | 2 |  |
| Initial tension Fo | 414 | N |
| Tangential force Ft | 518.66 | N |
| Shaft load Fr | 816.76 | N |
| Service life | 8000 | hours |

## CHAPTER III. DESIGN OF GEAR DRIVE

### 3.1	 Single stage helical reducer

#### 3.1.1 	Calculation on reducer shafts

Power of the driving wheel: PI = 2.763 kW

Power of the driven wheel: PII = 2.653 kW

Helical gear ratio: u = 6.3

Number of revolutions of driving wheel: nI = 1420 rpm

Number of revolutions of driven wheel: nII = 225.4 rpm

Torque on driving wheel: TI = 18.582 Nm

Torque on driven wheel: TII = 112.405 Nm

#### 3.1.2 	Select the driving and driven gear material

Following the Table 6.13 Contact and Bending Endurance Limits σOHlim and σOFlim:

Driving gear: choose C45 with hardness HB1 = 255 HB, heat treatment is

normalization, structural improvement.

Driven gear: choose C45 with hardness HB2 followed by formula H1 ≥ H2 +

(10 – 15) HB, so HB2 = 228 HB, heat treatment is normalization, structural

improvement.

#### 3.1.3	Determine the allowable contact stress [σH] and bending stress [σF]

Contact fatigue limit

+ For driving gear:

σOH lim1 = 2HB1 + 70 = 2×260 + 70 = 580 MPa

+ For driving gear:

σOH lim2 = 2HB2 + 70 = 2×228 + 70 = 526 MPa

Safety factor

SH1= SH2 = 1.1

Base number of cycle NHO

NHO1=30HB12.4 = 30×2552.4 = 17.9 × 106

NHO2=30HB22.4 = 30×2282.4 = 13.68 × 106

NFO1= NFO2= 5 × 106

Service life of the transmission system

LH = 8000 hours

The equivalent number of cycles if the gear is working with working load and

number of revolution n is constant:

NHE1=NFE1 = 60cn1LH = 60 × 1 × 1420 × 8000 = 681.6× 106

NHE2= NFE2= 60cn2LH = 60 × 1 × 225.4 × 8000 = 108.19 × 106

Life coefficient

KHL = 6NHONHE       and      KFL = 6NFONFE

Because:   NHE1 > NHO1; NHE2 > NHO2 so KHL1 = KHL2 = 1

and NFE1 > NFO1; NFE2 > NFO2  so KFL1 = KFL2 = 1

Allowable contact stress for the driving gear and driven gear

σH1=σOH lim10.9SHKHL1=580 ×0.91.1×1=474.55 MPa

σH2= σOH lim20.9SHKHL2=526 ×0.91.1×1=430.36 MPa

🡪 With helical spur gear, we have:

σH=0.5 ×(σH12+σH22)=0.5×(474.552+430.362)

= 453 MPa

Satisfied [σH]min ≤ [σH] ≤ 1.25[σH]min with [σH]min = [σH2]

Bending endurance limit

σOF lim = 1.8HB

σOF lim1 = 1.8HB1 = 1.8 × 255 = 459 MPa

σOF lim2 = 1.8HB2 = 1.8 × 228 = 410.4 MPa

Safety factor: SF = 1.75

Factor of the two-way rotation on fatigue strength: KFC = 1 when one-way rotation.

Allowable bending stress

[σF1] = σOF lim⁡1KFCKFL1SF = 4591 × 11.75 = 262.286 MPa

[σF2] = σOF lim⁡2KFCKFL2SF = 410.41 × 11.75 = 234.514 MPa

Allowable bending stress when overload

[σF1]max = 0.8σch1 = 646 MPa

[σF2]max = 0.8σch2 = 360 MPa

Allowable contact stress when overload:

[σH]max = 2.8σch = 1260 Mpa

#### 3.1.4	Select the gear width factor ψba according to the standard:

According to table 6.15, we have ψba=0.5

From here infer:  ψbd=ψba(u+1)2=0.5(6.3+1)2=1.825

According to table 6.4 Factors KHβ,KFβ, we choose KHβ=1.08 and KFβ=1.16

#### 3.1.5	The center distance aw:

Center distance aw:

aw=430u+1×3T1KHβψbaσH2u=4306.3+1×318.582×1.080.5×4532×6.3

=98.657 mm

According to the standard series we choose aw=100 mm

#### 3.1.6	The module m according to the center distance aw

mn = (0,01 ÷ 0,02)aw = (0,01 ÷ 0,02) × 100 = 1 ÷ 2 mm

We choose mn = 2 mm

#### 3.1.7	Total number of teeth

Because of the helical gear, we have:

8o≤β≤20o

cos 8o ≥mnz1(u+1)2aw≥ cos 20o

2awcos 8o mnu+1≥z1≥2awcos 20o mnu+1

2 ×100×cos 8o 26.3+1≥z1≥2 ×100×cos 20o 26.3+1

13.57 ≥z1≥12.87

We choose z1 = 13 teeth

So number of driving teeth z2 = z1u = z1 × 6.3 = 82 teeth

Ratio between face width and center distance

ψba = b2aw = 0.5

b2 = 50 mm and b1 = b2 + 5 = 55 mm

Tooth inclination angle

β = arccosm(z1 + z2)2aw= arccos2(13 + 82)2 ×100= 18.19o

#### 3.1.8	Recalculate the gear speed ratio u

u = z2z1 = 8213 = 6.308

Δu=6.308-6.32×100%=0.38% (<2÷3%)(accepted)

#### 3.1.9 	Geometry parameters of gear drive

Parameter of the pitch circle

- Driving gear: d1 = z1mncosβ = 13 × 2cos(18.19) = 27.37 mm

- Driven gear: d2 = z2mncosβ = 82 × 2cos(18.19) = 172.63 mm

Outside diameter

- Driving gear: da1 = d1 + 2mn = 27.37 + 2 × 2 = 31.37 mm

- Driven gear: da2 = d2 + 2mn = 172.63 + 2 × 2 = 176.63 mm

Root diameter

- Driving gear: df1 = d1 – 2.5mn = 27.37 - 2.5 × 2 = 22.37 mm

- Driven gear: df2 = d2 – 2.5mn = 172.63 - 2.5 × 2 = 167.63 mm

#### 3.1.10	Peripheral speed v and the angle of helical gear

v=πd1nI60000=π × 27.37 × 142060000=2.03 m/s

According to table 6.3 we choose grade of accuracy 9 with vgh = 6 m/s

αt = arctan tan20ocosβ=arctantan20ocos18.19o=20.96o

#### 3.1.11	The value of the forces acting on gear mesh

Peripheral force

Ft1=Ft2=2T1cosβmnz1=2×18.582×cos⁡(18.19o)2×13×103=1357.953 N

Axial force

Fa1=Fa2=Ft1tanβ=1357.953×tan 18.19o =446.210 N

Radial force

Fr1=Fr2=2Ft1tanαnwcosβ=2×1357.953×tan(20o)cos⁡(18.19o)=1040.507 N

The normal force

Fn1=Fn2=2Ft1cosβ×cosαt=2×1357.953cos 18.19o ×cos 20.96o =3061.336 N

#### 3.1.12	Dynamic load factor

According to table 6.6 Factors KHv and FFv, we choose

KHv=1.04 and FFv=1.09

According to table 6.11 Factors KHα, we choose KHα=1.13

KFα=1 (because ncx=9)

#### 3.1.13	Checking stress

Contact stress

σH=ZMZHZε2T1KH(u±1)dw12bwu≤σH

With σH :

σH=σOHlimKHLKLKVKlKxHSH

For helical gear we have σH:

σH=0.5 ×(σH12+σH22)=0.5×(474.552+430.362)

= 453 MPa

Where:

+ ZH=4cosβsin⁡(2αtw)=4×cos 18.19o sin⁡(2×20.96o)=2.385

+ Zε=1εα=0.812

Where εα=1.88-3.21z1+1z2cosβ=1.88-3.2113+182cos 18.19o

=1.515>1

+ ZM=190 MPa1/2 (because both driving and driven gear are C45 steel)

+ KH=FHβ×FHv×FHα=1.08×1.04×1.13=1.269

σH=ZMZHZε2T1KHu+1dw12bwu

=190×2.385×0.812×2×18.582×103×1.269×6.3+127.372×50×6.3

=444.448 MPa<453 MPa

As σH<[σH] so the gear has enough contact strength

Equivalent gear teeth

zv1=z1cos3β=13cos3(18.19o)=15 teeth

zv2=z2cos3β=82cos3(18.19o)=96 teeth

Bending stress

[σF1] = 262.286 MPa

[σF2] = 234.514 Mpa

σF=YFFTKFYεYβbwmn

Where:

+ KF=FFβ×FFv×FFα=1.16×1.09×1=1.264

+ Yε=1εα=11.515=0.66

+ Yβ=1-β140=1-18.19140=0.87

+ FT=2T1cosβmnz1=2×18.582×cos⁡(18.19o)2×13×103=1357.953

+ YF1=3.47+13.2zv1-27.9xzv1+0.092x2=3.47+13.215=4.35 (x=0)

+ YF2=3.47+13.2zv2-27.9xzv2+0.092x2=3.47+13.296=3.6075 (x=0)

[σF1] YF1=262.286 4.35=60.296                                      [σF2] YF2=234.514 3.6075=65.007

Because [σF1] YF1<[σF2] YF2so we choose driving gear to calculate

σF1=YF1FTKFYεYβbwmn=4.35×1357.953×1.264×0.66×0.8780×2

=26.796 MPa<262.286 MPa

As σF1<σF1 so the gear has enough bending strength

### 3.2	 Open gear drive

#### 3.2.1 	Calculations of transmission in gear drive

Power of the driving wheel: PIII = 2.364 kW

Power of the driven wheel (drying drum): PWorking = 2.2 kW

Straight spur gear ratio u = 10

Number of revolutions of driving wheel: nIII = 199.47 rpm

Number of revolutions of driven wheel: ndrum = 19.95 rpm

Torque on driving wheel TIII = 113.181 Nm

Torque on driven wheel Tdrum = 1053.133 Nm

#### 3.2.2	Select the driving and driven gear material

Following the Table 6.13 Contact and Bending Endurance Limits σOHlim and σOFlim:

Driving gear: choose C45 with hardness HB3 = 195 HB, heat treatment is

normalization, structural improvement.

Driven gear: choose C45 with hardness HB4 followed by formula H3 ≥ H4 +

(10 – 15) HB, so HB4 = 180 HB, heat treatment is normalization, structural

improvement.

#### 3.2.3 	Determine the allowable contact stress [σH]  bending stress [σF]

Contact fatigue limit

+ For driving gear:

σOH lim3 = 2HB3 + 70 = 2×195 + 70 = 460 MPa

+ For driving gear:

σOHl lim4 = 2HB4 + 70 = 2×180 + 70 = 430 MPa

Safety factor

SH3= SH4 = 1.1

Base number of cycle NHO

NHO3=30HB32.4 = 30×2002.4 = 9.402 × 106

NHO4=30HB42.4 = 30×1802.4 = 7.758× 106

NFO3= NFO4= 5 × 106

Service life of the transmission system

LH = 8000 hours

The equivalent number of cycles if the gear is working with working load and

number of revolution n is constant:

NHE3=NFE3 = 60cn3LH = 60 × 1 × 199.47×8000 = 95.75 × 106

NHE4= NFE4= 60cn4LH = 60 × 1 × 19.95 ×8000 = 9.58 × 106

Life coefficient

KHL = 6NHONHE       and      KFL = 6NFONFE

Because:   NHE3 > NHO3; :   NHE4 > NHO4; NFE3 > NFO3; NFE4 > NFO4

so KHL3=KHL4=1 and KFL3=KFL4=1

Allowable contact stress for the driving gear and driven gear

σH3=σOH lim30.9SHKHL3=460 ×0.91.1×1=376.364 MPa

σH4= σOH lim40.9SHKHL4=430 ×0.91.1×1=351.818 MPa

With straight spur gear, we have:

σH=MinσH3,σH4=351.818 MPa

Bending endurance limit

σOF lim = 1.8HB

σOF lim3 = 1.8HB = 1.8 × 195 = 351 MPa

σOF lim4 = 1.8HB = 1.8 × 180 = 324 MPa

Safety factor: SF = 1.75

Factor of the two-way rotation on fatigue strength: KFC = 1 when one-way rotation.

Allowable bending stress

[σF3] = σOF lim⁡3KFCKFL3SF = 351 ×1 × 11.75 = 200.571 MPa

[σF4] = σOF lim⁡4KFCKFL4SF = 324 ×1 × 11.75 = 185.143 MPa

#### 3.2.4 	Total number of teeth

Choose the number of teeth of driving gear z3=20 teeth

Therefore, z4=u×z3=10×20=200 teeth

#### 3.2.5 	Calculating YF factor:

+ For driving gear:

YF3=3.47+13.2z3-27.9xz3+0.092x2=3.47+13.220=4.13 (x=0)

+ For driven gear:

YF4=3.47+13.2z4-27.9xz4+0.092x2=3.47+13.2200=3.536 (x=0)

[σF3] YF3=200.571 4.13=48.565                                    [σF4] YF4=234.514 3.536=52.359

Because [σF3] YF3<[σF4] YF4so we choose driving gear to calculate

#### 3.2.6 	Select the gear width factor ψbd according to the standard:

According to table 6.16, we have ψbd=0.8

According to table 6.4 Factors KHβ,KFβ, we choose KFβ=1.05

#### 3.2.7 	The module m according to the bending stress [σF]

m=14×3T3KFβYFz32ΨbdσF=14×3113.181×1.05×4.13202×0.8×200.571

=2.758 mm

As standard, we choose m = 3 mm

#### 3.2.8 	Geometry parameters of gear drive:

Parameter of the pitch circle

- Driving gear: d3 = z3m=20×3=60 mm

- Driven gear: d4 = z4m=200×3=600 mm

Outside diameter

- Driving gear: da3 = d3+2m=60+2×3=66 mm

- Driven gear: da4 = d4+2m=600+2×3=606 mm

Root diameter

- Driving gear: df3 = d3-2.5m=60-2.5×3=52.5 mm

- Driven gear: df4 = d4-2.5m=600-2.5×3=592.5 mm

The center distance

aw=m(z3+z4)2=3×(20+200)2=330 mm

Face width

- Driven gear: b4 = ψbdd3=0.8×60=48 mm

- Driving gear: b3 = b4+5=48+5=53 mm

#### 3.2.9 	Peripheral speed v:

v = πd3n360000 = π × 60 × 199.4760000 = 0.627 m/s

According to table 6.3 we choose grade of accuracy 9 with vgh = 3 m/s

#### 3.2.10	The value of the forces acting on gear mesh

Peripheral force

Ft3=2T3mz3=2×113.181 3×20×103=3772.7 N

Radial force

Fr3=Ft3tan αw=3772.7×tan20o=1373.151 N

The normal force

Fn3=Ft3cos αw=3772.7cos 20o =4014.823 N

#### 3.2.11	Dynamic load factor

According to table 6.5 Factors KHV and FFV, we choose

KHV=1.06 and FFV=1.11

#### 3.2.12	Checking bending stress

Because [σF3] YF3<[σF4] YF4so we choose driving gear to calculate

Therefore, σF3=2YF3T3KFβKFVdw3bwmn=2×103×4.13×113.181×1.05×1.1160×48×3

=126.111 MPa ≤200.571 MPa

#### 3.3	 Summary

|  | Parameter |  | Value |
| --- | --- | --- | --- |
| Single stage helical reducer | The pitch diameter | Driving wheel | d1=27.37 (mm) |
|  |  | Driven wheel | d2= 172.63 (mm) |
|  | The root diameter | Driving wheel | df1=22.37 mm |
|  |  | Driven wheel | df2= 167.63 mm |
|  | The pitch outside diameter | Driving wheel | da1=31.37 (mm) |
|  |  | Driven wheel | da2=176.63 (mm) |
|  | Number of teeth | Driving wheel | 13 |
|  |  | Driven wheel | 82 |
| Open gear drive | The pitch diameter | Driving wheel | d1=60 (mm) |
|  |  | Driven wheel | d2=600 (mm) |
|  | The root diameter | Driving wheel | df1=52.5 mm |
|  |  | Driven wheel | df2= 592.5 mm |
|  | The pitch outside diameter | Driving wheel | da1=66 (mm) |
|  |  | Driven wheel | da2=606(mm) |
|  | Number of teeth | Driving wheel | 20 |
|  |  | Driven wheel | 200 |

## CHAPTER IV. CALCULATE AND DESIGN THE SHAFT

Ft1=Ft2=811.689  NFr1=Fr2=599.306 N Fa1=Fa2=137.724 NFx=1579 NFt3=3772.7 NFr3=1373.151 N ; TI=18.582 NmTII=112.405 NmTIII=113.181 Nm

4.1	Choose material and allowable stresses:

We choose the same material with the gear C45 for the shaft I with hardness 270 HB , heat treatment is normalization, structural improvement with the allowable stress and the allowable torsion:

σb1=883 MPa ;σch1=638 MPa; τch1= 383 MPa, σ1=85 MPa

We choose the same material with the gear C45 for the shaft II with hardness 245 HB , heat treatment is normalization, structural improvement with the allowable stress and the allowable torsion:

σb2=785 MPa ;σch2=540 MPa; τch2= 324 MPa, σ2=70 MPa

Preliminary choosing allowable torsional stress τ=20 MPa for shaft 1, 3 and τ=25 MPa for shaft 2

d1≥10×3T10.2τ1=10×318.582 0.2×20=16.69 mm

d2≥10×3T20.2τ2=10×3112.4050.2×25=28.22 mm

d3≥10×3T20.2τ3=10×3113.1810.2×20=30.47 mm

Select preliminary diameter of shaft I: d1=20 mm

Select preliminary diameter of shaft II: d2=30 mm

Select preliminary diameter of shaft II: d3=32 mm

4.2	Select the length of shaft I:

- Select preliminary diameter of shaft I: dI=30 (mm).

- Select the smallest diameter of shaft I at the position of the belt with d1=36 (mm).

- The preliminary diameter at the bearing: d1=45 (mm)

- The preliminary diameter at the oil seal ring: d2=50 (mm)

- The preliminary diameter at the gear: d2=55 (mm)

- l=b+2×e1+w=185+2×10+35=240 (mm)

- f=120 mm

4.3	Calculate the reaction forces acting on shaft I:

SHAFT I

Frb

Frb

Fa1

Fr1

Ft1

Ft1

Fr1

RAy

RBy

RBx

RAx

Ma1

{Ft1=Ft2=811.689  NFr1=Fr2=599.306 N  Fa1=Fa2=137.724 N Frb=816.76 N  ; { TI=82.329 Nm TII=197.35 Nm

Moment caused by axial force:

Ma1 = Fa1 ×d12 = 137.724×202.86×10-32 =13.97 Nm

The torque acting on the shaft:

T1 =Ft1 ×d12=811.689×202.86×10-32=82.33 Nm

The moment equilibrium equation in y-z plane at location A:

∑MA=Frb×360×10-3-RBy1×240×10-3+Fr1×120×10-3+Ma1=0

816.76×360×10-3-RBy1×240×10-3+599.306×120×10-3+13.97=0

RBy1=1583 N

The force equilibrium equation on y axis is:

∑F=RBy1-Frb-RAy1-Fr1=0

RAy1=1583-816.76-599.306

RAy1=166.934 N

The moment equilibrium equation in x-z plane at location A:

∑MA=-RBx1×240×10-3+Ft1×120×10-3= 0

-RBx1×240×10-3+811.689×120×10-3= 0

RBx1=405.84 N

The force equilibrium equation on x axis is:

∑F=RBx1+RAx1-Ft1=0

RAx1=811.689-405.84

RAx1=405.84 N

4.4 Diagram of bending moment and torque diagram of shaft I:

Fa1

Fr1

Ft1

Frb

RAy

RAx

Frb

RBy

RBx

Fr1

Ma1

Ft1

RBy

Fr1

RAy

Frb

Ma1

98.01

Mx (Nm)

20.03

6.06

Ft1

RBx

RAx

My (Nm)

48.7

82.329

T (Nm)

According to moment distribution diagram, the most critical position is section B:

MtdB1=MBx12+MBy12+0.75×T12=98.012+02+0.75×82.3292

=121.2 Nm

Shaft I is made of Normalized 45-steel ⇒ Allowable bending stress [𝜎] = 85 𝑀𝑃a

Shaft diameter at section B is determined:

dB1≥10×332×MtdB1π×σ1=10×332×121.2 π×85=24.4 (mm)

According to the standard we can choose d = 45 mm

Calculation for static strength:

σb1=Mcritical10.1×d3  [σb]

σb1=121.20.1×(45×10-3)3=13.3 MPa    σb=100 MPa (satisfied)

Equivalent moment at C, D:

MtdC1=MCx12+MCy12+0.75×T12=20.032+48.72+0.75×82.3292=88.64 Nm

MtdD2=MDx12+MDy12+0.75×T12=02+02+0.75×82.3292=71.3 Nm

Shaft diameter at section C is determined:

dC1≥10×332×MtdC1π×σ1=10×332×88.64π×85=21.98(mm)

Because at position C there is a keyway, the 5% increase is 23.08 mm, and according to the standard we can choose d = 55 mm.

Shaft diameter at section D is determined:

dD1≥10×332×MtdD1π×σ1=10×332×71.3π×85=20.44 (mm)

Because at position D there is a keyway, the 5% increase is 21.47 mm, and according to the standard we can choose d = 36 mm.

4.5 Calculations and design preliminary parameter for the shaft II

- Select preliminary diameter of shaft II: dII=40 (mm).

- The preliminary diameter at the bearing: d1=50 (mm).

- The preliminary diameter at the oil seal ring: d2=55 mm

- The preliminary diameter at the helical gear: d2=60 mm

- The preliminary diameter at the straight spur gear: d3=45 mm

- l=b+2×e1+w=180+2×10+60=260 (mm)

- f=100 mm

4.6 Calculate the reaction forces acting on shaft II:

Shaft II

Fr2

Ft2

Fa2

Ft3

Fr3

Fr3

Ft3

RBy

RBx

RAy

RAx

Fr2

Ft2

Ma2

Moment caused by axial force:

Ma2 = Fa2 ×d22 = 137.724  × 507.15×10-32 =34.92 Nm

The torque acting on the shaft:

T2=Ft2 ×d22=811.689×507.15×10-32=197.345 Nm

The moment equilibrium equation in y-z plane at location A:

∑MA=RBy2×260×10-3+Fr2×100×10-3-Fr3×360×10-3-Ma2=0

RBy2×260×10-3+599.306×100×10-3-897.85×360×10-3-34.92=0

RBy2=943.66 N

The force equilibrium equation on y axis is:

∑Fy=RAy2-RBy2-Fr2+Fr3=0

RAy2=943.66+599.306-897.85

RAy2=645.12 N

The moment equilibrium equation in x-z plane at location A:

∑Mx/A =Ft3×360×10-3-RBx2×260×10-3+Ft2×130×10-3= 0

2466.81×360×10-3-RBx2×260×10-3+811.689×130×10-3= 0

RBx2=3821.43 N

The force equilibrium equation on x axis is:

∑Fx=Ft3+RAx2-RBx2+Ft2=0

RAx2=2466.81-3821.43+811.689

RAx2=542.93 N

4.7	Diagram of bending moment and torque diagram of shaft II:

Mx (Nm)

My (Nm)

T (Nm)

Fr3

Ft3

Fr2

Ft2

Fa2

Fr3

Ft3

RBy

RBx

RAy

RAx

Fr2

Ft2

Ma2

Fr3

RBy

RAy

Fr2

Ma2

Ft3

RBx

RAx

Ft2

89.785

246.681

83.87

48.95

70.58

197.345

According to moment distribution diagram, the most critical position is section B:

MtdB2=MBx22+MBy22+0.75×T22

=89.7852+246.6812+0.75×197.352

=313.25 Nm

Shaft II is made of Normalized 45-steel ⇒ Allowable bending stress [𝜎] = 70 𝑀𝑃a

Shaft diameter at section B is determined:

dB2≥10×332×MtdB2π×σ2=10×332×313.25 π×70=35.72 (mm)

According to the standard we can choose d = 50 mm

Calculation for static strength:

σb2=Mcritical20.1×d3  [σb]

σb2=313.250.1×(50×10-3)3=25.1 MPa    σb=100 MPa (satisfied)

Equivalent moment at C, D:

MtdD2=MDx22+MDy22+0.75×T22=02+02+0.75×197.3452

=170.91 Nm

MtdC2=MCx22+MCy22+0.75×T22=83.872+70.582+0.75×197.3452

=203.04 Nm

Shaft diameter at section D is determined:

dD2≥10×332×MtdD2π×σ2=10×332×170.91 π×70=29.19 (mm)

Because at position D there is a keyway, the 5% increase is 30.649 mm, and according to the standard we can choose d = 45 mm

Shaft diameter at section C is determined:

dC2≥10×332×MtdC2π×σ2=10×332×203.04 π×70=30.91 (mm)

Because at position C there is a keyway, the 5% increase is 32.46 mm, and according to the standard we can choose d = 60 mm

4.8 Calculations and design preliminary parameter for the shaft III

- Select preliminary diameter of shaft III: dIII=32 (mm).

- The preliminary diameter at the chain: d2=32 (mm)

- The preliminary diameter at the bearing: d1=35 (mm).

- The preliminary diameter at the oil seal ring: d2=45 (mm)

- The preliminary diameter at the straight spur gear: d3=40 (mm)

- l=b+2×e1+w=50+2×10+50=120 (mm)

- f=60 mm

4.6 Calculate the reaction forces acting on shaft II:

Shaft II

Fx

Fx

Ft3

RBx

RBy

Fr3

RAx

RAy

Fr3

Ft3

The torque acting on the shaft:

T3=113.181 Nm

The moment equilibrium equation in y-z plane at location A:

∑MA=Fx×180×10-3-RBy3×120×10-3+Fr3×60×10-3=0

1579×180×10-3-RBy3×120×10-3+1373.151×60×10-3=0

RBy3=3055.075 N

The force equilibrium equation on y axis is:

∑F=RBy1-Fx-RAy3-Fr3=0

RAy3=3055.075-1579-1373.151

RAy1=102.925 N

The moment equilibrium equation in x-z plane at location A:

∑MA=-RBx3×120×10-3+Ft3×60×10-3= 0

-RBx3×120×10-3+3772.7×60×10-3= 0

RBx3=1886.35 N

The force equilibrium equation on x axis is:

∑F=RBx3+RAx3-Ft3=0

RAx1=3772.7-1886.35

RAx1=1886.35 N

4.7	Diagram of bending moment and torque diagram of shaft II:

113.181

T (Nm)

My (Nm)

Mx (Nm)

6.176

113.181

FAx

RBx

Ft3

Fr3

RAy

Fx

Fx

Fx

94.74

RBy

RBx

RAx

RAy

Fr3

Ft3

RBy

Ft3

Fr3

According to moment distribution diagram, the most critical position is section C:

MtdC3=MCx32+MCy32+0.75×T32

=6.1762+113.1812+0.75×113.1812

=149.852 Nm

Shaft III is made of Normalized C45-steel, Allowable bending stress [𝜎] = 70 𝑀𝑃a

Shaft diameter at section C is determined:

dC3≥10×332×MtdC3π×σ2=10×332×149.852 π×70=27.938 (mm)

Because at position D there is a keyway, the 5% increase is 29.335 mm, and according to the standard we can choose d = 40 mm

Calculation for static strength:

σb2=Mcritical20.1×d3  [σb]

σb2=149.8520.1×(40×10-3)3=23.41 MPa   <σb=100 MPa (satisfied)

Equivalent moment at B, D:

MtdD3=MDx32+MDy32+0.75×T32=02+02+0.75×113.1812

=98.018 Nm

MtdB3=MBx32+MBy32+0.75×T32=94.742+02+0.75×113.1812

=136.32 Nm

Shaft diameter at section D is determined:

dD2≥10×332×MtdD2π×σ2=10×332×98.018 π×70=24.251 (mm)

Because at position D there is a keyway, the 5% increase is 25.464 mm, and according to the standard we can choose d = 30 mm

Shaft diameter at section B is determined:

dB3≥10×332×MtdB3π×σ2=10×332×136.32 π×70=27.07 (mm)

According to the standard we can choose d = 35 mm

4.8	Summary the diameter:

Shaft I

|  | A (Bearing) | B (Bearing) | C (gear) | D (belt) |
| --- | --- | --- | --- | --- |
| d(mm) | 45 | 45 | 55 | 36 |

Shaft II

|  | A (Bearing) | B (Bearing) | C (Helical gear) | D (Straight spur gear) |
| --- | --- | --- | --- | --- |
| d(mm) | 50 | 50 | 60 | 45 |

Shaft III

|  | A (Bearing) | B (Bearing) | C (Straight spur gear) | D (Chain) |
| --- | --- | --- | --- | --- |
| d(mm) | 35 | 35 | 40 | 30 |

4.9	Choose the Preliminary key for the shaft:

- We choose the same material with the gear C45, heat treatment is normalization, structural improvement.

- For the shaft I, we have the key at the position of the belt and the gear with the diameter of each part:

{d belt=36 mm d gear=55 mm

- We choose the key at the belt with the standard parameter:

{b×h×l=10×8×50 mm t1=5 mm;t2=3.3 mm

- We choose the key at the gear with the same width at the belt:

{b×h×l=10×8×50 mm t1=5 mm;t2=3.3 mm

- Calculation with the crushing condition for the key at the gear:

σd=Ft2×ll=2993.783.2×55=17.01 MPa≤σd(MPa)

With:

+ ll=l=50 (mm): the working length of the key.

+  t2=0.4×h=0.4×h=3.2 (mm): the depth of the keyway.

+ F=2×T×103d=2×82.329×10355=2993.78 (N): the tangential force acting on the shaft.

+  [σd ] = 150 MPa: Allowable volume stress (based on table 9.5 in the reference [1], the key is C45 and the system is static load).

- Calculation:

τc=Fb×ll≤[τc]

- At the belt:

τc=4573.8310×50=9.15 (MPa)≤[τc]

- At the gear:

τc=2993.78 10×50 =5.99 (MPa)≤[τc]

With:

+ ll=l=50 (mm): the working length of the key.

+ b=10 (mm): the width of the key.

+ F=2×T×103d=2×82.329×10355=2993.78  (N): the tangential force at the gear.

+ F=2×T×103d=2×82.329×10336=4573.83 (N): the tangential force at the belt.

+ [τc ] = 120 MPa: Allowable shear stress (the key is C45 and the system is static load.).

- For the shaft II, we have the key at the position of the helical gear and the straight spur gear with the diameter of each part:

{d straight=45mm d  helical=60 mm

- We choose the key at the straight spur gear with the standard parameter:

{b×h×l=14×9×56 mm t1=5.5 mm;t2=3.8 mm

- We choose the key at the helical gear with the same width at the straight spur gear:

{b×h×l=14×9×56 mm t1=5.5 mm;t2=3.8 mm

- Calculation with the crushing condition for the key at the helical gear in the reducer:

σd=Ft2×ll=4933.63  3.8×56=23.18 MPa≤σd

With:

+ ll=l=56 (mm): the working length of the key.

+  t2=0.4×h=3.6 (mm): the depth of the keyway.

+ F=2×T×103d=2×197.345×10360=6578.17 N: the tangential force acting on the shaft.

+  [σd ] = 150 MPa: Allowable volume stress (the key is C45 and the system is static load).

- Calculation:

τc=Fb×ll≤[τc]

- At the straight spur gear:

τc=8770.89 14 ×56 =11.19 (MPa)≤[τc]

- At the helical gear:

τc=6578.17  14 ×56    =8.39 (MPa)≤[τc]

With:

+ ll=l=56 (mm): the working length of the key.

+ b=14 (mm): the width of the key.

+ F=2×T×103d=2×197.345×10345=8770.89 (N): the tangential force at the straight spur gear.

+F=2×T×103d=2×197.345×10360=6578.17 (N): the tangential force at the helical gear.

+ [τc ] = (60 ÷ 90) MPa: Allowable shear stress (the key is C45 and the system is static load.).

4.10  Checking the shaft according to the safety factor:

σb=Mcritical0.1×d3  [σb]

- Calculation for static strength for the shaft I:

σb1=121.20.1×(55×10-3)3=7.3 MPa    σb=100 MPa (satisfied)

- Calculation for static strength for the shaft II:

σb2=313.25 0.1×(60×10-3)3=14.5 MPa    σb=100 MPa (satisfied)

- Calculation for the fatigue strength and the equivalent stress when overload:

s=sσsτsσ2+sτ2≥s=2.5

- With

sσ=σ-1Kσσaεσβ+ѱσσm          ;         sτ=τ-1Kττaετβ+ѱττm

- Where:

+ For shaft I: σb=883 MPa ;σch=638 MPa; τch= 383 MPa, σ=85 MPa

+ σ-1=0.4×σb=353.2 MPa ; τ-1=0.25×σb=220.75 MPa

+ For shaft II: σb=785 MPa ;σch=540 MPa; τch= 324 MPa, σ=70 MPa

+ σ-1=0.4×σb=314 MPa ; τ-1=0.25×σb=196.25 MPa

+ ѱσ,ѱτ: based on the figure 2.11 - Giáo trình Cơ sở thiết kế máy:

ѱσ=0.1 ; ѱτ=0.05.

+ εσ, ετ: based on table 10.4 - Giáo trình Cơ sở thiết kế máy:

εσ-I=0.81; ετ-I=0.76; εσ-II=0.75; ετ-II=0.73

+ β=1.7

+ Kσ,Kτ: Based on table 10.8 - Giáo trình Cơ sở thiết kế máy: Kσ=2.05; Kτ=1.9

Calculate  Mtd=Mx2+My2

On shaft I:

MtdA1=MtdD1= 0 (Nm)

MtdB1=98.012+02=98.01 (Nm)

MtdC1 =20.032+48.72=52.66 (Nm)

On shaft II:

MtdA2=MtdD2=0 (Nm)

MtdB2=89.7852+246.6812=262.51 (Nm)

MtdC2=83.872+70.582=109.62 (Nm)

Calculate W=πd332-btd-t22d or W=πd332

On shaft I:

WA1=π×45332=8946.18×10-9 m3

WB1=π×45332=8946.18×10-9 m3

WC1=π×55332-10×5.0×55-5.022×55=15197.46×10-9 m3

WD1=π×36332-10×5.0×36-5.022×36=3913.08×10-9 m3

On shaft II:

WA2=π×50332=12271.85×10-9 m3

WB2=π×50332=12271.85×10-9 m3

WC2=π×60332-14×5.5×60-5.522×60=19299.84×10-9 m3

WD2=π×45332-18×5.5×45-5.522×45=7611.3×10-9 m3

Calculate Wo=πd316-btd-t22d or Wo=πd316

On shaft I:

WoA1=π×45316=17892.35×10-9 m3

WoB1=π×45316=17892.35×10-9 m3

WoC1=π×55316-10×5.0×55-5.022×55=31531.29×10-9 m3

WoD1=π×36316-10×5.0×36-5.022×36=8493.52×10-9 m3

On shaft II:

WoA1=π×50316=24543.69×10-9 m3

WoB1=π×50316=24543.69×10-9 m3

WoC1=π×60316-14×5.5×60-5.522×60=40505.59×10-9 m3

WoD1=π×45316-14×5.5×45-5.522×45=16557.47×10-9 m3

Calculate σa=M×103W

On shaft I:

σaA1=σaD1=0 MPa

σaB1=98.01×1038946.18=10.96 MPa

σaC1=52.66×10315197.46=3.47 MPa

On shaft II:

σaA2=σaD2=0 MPa

σaB2=262.51×10312271.85=21.39 MPa

σaC2=109.62 ×10319299.84=5.68 MPa

Calculate τa=τm=T×1032Wo

On shaft I:

τaA1=0 MPa

τaB1=82.329×103 2×17892.35=2.3 MPa

τaC1=82.329×1032×31531.29=1.31 MPa

τaD1=82.329×1032×8493.52=4.85 MPa

On shaft II:

τaA2=0 MPa

τaB2=197.345×1032×24543.69=4.02 MPa

τaC2=197.345×1032×40505.59=2.44 MPa

τaD2=197.345×1032×16557.47=5.96 MPa

Calculate sσ=σ-1Kσσaεσβ+ѱσσm

On shaft I:

sσB1=353.22.05×10.96 0.81×1.7+0.1×0=21.65

sσC1=353.22.05×3.470.81×1.7+0.1×0=68.37

On shaft II:

sσB2=220.752.05×21.39 0.75×1.7+0.1×0=6.42

sσC2=220.752.05×5.680.75×1.7+0.1×0=24.17

Calculate sτ=τ-1Kττaετβ+ѱττm

On shaft I:

sτB1=3141.9×2.30.76×1.7+0.05×0=92.83

sτC1=3141.9×1.310.76×1.7+0.05×0=162.99

sτD1=3141.9×4.75 0.76×1.7+0.05×0=44.95

On shaft II:

sτB2=196.251.9×4.02 0.73×1.7+0.05×0=31.89

sτC2=196.251.9×2.440.73×1.7+0.05×0=52.53

sτD2=196.251.9×5.960.73×1.7+0.05×0=21.51

Calculate s=sσsτsσ2+sτ2

On shaft I:

sB1=21.65×92.8321.652+92.832=21.08

sC1=68.37×162.9968.372+162.992=63.05

On shaft II:

sB2=6.42×31.896.422+31.892=6.29

sC2=24.17×52.5324.172+52.532=21.96

Table For Summary

Shaft I:

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| W(m3) | 8946.18×10-9 | 8946.18×10-9 | 15197.46×10-9 | 3913.08×10-9 |
| Wo (m3) | 17892.35×10-9 | 17892.35×10-9 | 31531.29×10-9 | 8493.52×10-9 |
| σa(MPa) | 0 | 10.96 | 3.47 | 0 |
| τa=τm(MPa) | 0 | 2.3 | 1.31 | 4.85 |
| sσ | ⎯ | 21.65 | 68.37 | ⎯ |
| sτ | ⎯ | 92.83 | 162.99 | 44.95 |
| s | ⎯ | 21.08>2.5 | 63.05>2.5 | ⎯ |

Shaft I satisfied the fatigue strength condition and flexural rigidity

Shaft II:

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| W(m3) | 12271.85×10-9 | 12271.85×10-9 | 19299.84×10-9 | 7611.3×10-9 |
| Wo (m3) | 24543.69×10-9 | 24543.69×10-9 | 40505.59×10-9 | 16557.47×10-9 |
| σa(MPa) | 0 | 21.39 | 5.68 | 0 |
| τa=τm(MPa) | 0 | 4.02 | 2.44 | 5.96 |
| sσ | ⎯ | 6.42 | 24.17 | ⎯ |
| sτ | ⎯ | 31.89 | 52.53 | 21.51 |
| s | ⎯ | 6.29>2.5 | 21.96>2.5 | ⎯ |

Shaft II satisfied the fatigue strength condition and flexural rigidity

4.11 Check the equivalent stress when overload

σtđ=σ2+3τ2≤σqt≈0.8×σch (MPa)

+ For the shaft I: σqt≈0.8×σch=510.4 (MPa)

+ For the shaft II: σqt≈0.8×σch=432 (MPa)

On shaft I:

σtđA=0 MPa

σtđB=10.962+3×2.32=11.66 MPa

σtđC=3.47+3×1.312=4.15 MPa

σtđD=02+3×4.852=8.4 MPa

On shaft II:

σtđA=0 MPa

σtđB=21.392+3×4.022=22.5 MPa

σtđC=5.682+3×2.442=7.08  MPa

σtđD=02+3×5.96 2=10.32 MPa

| Shaft |  | σ | τ | σtđ |
| --- | --- | --- | --- | --- |
| I | A | 0 | 0 | 0 |
|  | B | 10.96 | 2.3 | 11.66 |
|  | C | 3.47 | 1.31 | 4.15 |
|  | D | 0 | 4.85 | 8.4 |
| II | A | 0 | 0 | 0 |
|  | B | 21.39 | 4.02 | 22.5 |
|  | C | 5.68 | 2.44 | 7.08 |
|  | D | 0 | 5.96 | 10.32 |

Shaft I and II satisfied the condition.

## CHAPTER V. CALCULATE AND DESIGN THE BEARING

5.1 Calculations and design the bearing in shaft I

Shaft I

- Number of revolutions on shaft I: nI=450.07 (rpm)

- The preliminary diameter at the bearing: d1=45 (mm)

- Service life of the transmission system: Lh=8000 (h)

- Axial force: Fa1=137.724 N

- The centrifugal force acting on the bearing at A and B:

FrA1=RA1=RAx12+RAy12=405.842 +166.9342=438.83 (N)

FrB1=RB1=RBx12+RBy12=405.842+15832=1634.2 (N)

Due to FrB1>FrA1 so we chose FrB1

Fa1FrB1=137.724 1634.2=0.08

This satisfies the condition that deep groove ball bearing is suggested: 0<FaFr≤0.7

Select single row deep groove ball bearing bearings

Based on Schedule 9.1 in reference [55] we have:

| Sign | d, mm | D, mm | B, mm | r, mm | C, kN | C0, kN |
| --- | --- | --- | --- | --- | --- | --- |
| 700109 | 45 | 75 | 10 | 0.5 | 10.5 | 8.57 |

Fa1C0=137.724 8570=0.016

So e=0.19

We have: FaV(FrB)=137.7241634.2=0.08<e=0.19,

Based on the table 11.3 - Giáo trình Cơ sở thiết kế máy, X=1, Y=0

With V=1

We calculate the load at B with the formula:

Q=Qr=(XVFr+YFa)KtKσ

Where:

+Kσ: Coefficient of load specificity Kσ=1

+Kt: Coefficient of temperature Kt=1

+V: Coefficient of which ring rotate, V=1 if internal ring rotate

Q=Qr=XVFr+YFaKtKσ

=1×1×1634.2+0×137.724  ×1×1

=1634.2 N

- Longevity of bearing in million rounds:

L=60Lhn106=60×8000×450.07106=216.03 (million revolutions)

- The dynamic load sustainability:

Ctt=QL1m=1634.2×216.03 13=9805.65 N <C=10500 N

With: m = 3: Grade of bending line for ball bearing.

=> The condition is satisfied

- Re-calculate the longevity of bearing:

Q3L=C3 ->L=105001634.23=265.25 (million revolutions)

- Service life of the bearing:

Lh=106L60n=106×265.2560×450.07=9822.48 (h)

- Test for static load:

Q0=X0×FrB1+Y0×Fa=0.6×1634.2+0.5×137.724=1049.38 (N) and Q0=FrB1=1634.2 (N)

Where:

Q0: Static notational load.

C0: Allowable static notational load..

X0: Coefficient of centripetal load X0=0.6 (based on table 11.6 - Giáo trình Cơ sở thiết kế máy).

Y0: Coefficient of axial load Y0=0.5 (based on table 11.6 - Giáo trình Cơ sở thiết kế máy).

Because Q0=1634.2 N<C0=8570N  so condition is satisfied.

- Limited number of revolutions of bearing:

Dpwn=const

Where: Dpw: Center diameter of rollers, Dpw=d+D2=45+752=60 (mm)

n: Number of revolutions

Based on Table 11.7 - Giáo trình Cơ sở thiết kế máy, we have:

Dpwn=4.5×105 (mm.rpm) with deep groove ball bearing lubricated with grease.

ngh=4.5×10560=7500 (rpm)

Because n<ngh, condition is satisfied.

5.2 Calculations and design the bearing in shaft II

Shaft II

{Ft1=Ft2=811.689  NFr1=Fr2=599.306 N  Fa1=Fa2=137.724 N Frb=816.76 N Ft3=2466.81 N Fr3=897.85 N   ; { TI=82.329 Nm TII=197.345 Nm

- Number of revolutions on shaft I: nII=180.02 (rpm)

- The preliminary diameter at the bearing: d2=50 (mm)

- Service life of the transmission system: Lh=8000 (h)

- Axial force: Fa2=137.724 N

- The centrifugal force acting on the bearing at A and B:

FrA2=RA2=RAx22+RAy22=542.932 +645.122=843.18 (N)

FrB2=RB2=RBx22+RBy22=3821.432+943.662=3936.22 (N)

Due to FrB2>FrA2 so we chose FrB2

Fa2FrB2=137.7243936.22=0.035

This satisfies the condition that deep groove ball bearing is suggested: 0<FaFr≤0.3

Select single row deep deep groove ball bearing

Based on Schedule 9.1 in reference [55] we have:

| Sign | d, mm | D, mm | B, mm | r, mm | C, kN | C0, kN |
| --- | --- | --- | --- | --- | --- | --- |
| 110 | 50 | 90 | 20 | 2 | 27.5 | 20.2 |

Fa2C0=137.724 20200=0.007

So e=0.19

We have: FaV(FrB)=137.7243936.22=0.035<e=0.19,

Based on the table 11.3 - Giáo trình Cơ sở thiết kế máy, X=1, Y=0

With V=1

We calculate the load at B with the formula:

Q=Qr=(XVFr+YFa)KtKσ

Where:

+Kσ: Coefficient of load specificity Kσ=1

+Kt: Coefficient of temperature Kt=1

+V: Coefficient of which ring rotate, V=1 if internal ring rotate

Q=Qr=XVFr+YFaKtKσ

=1×1×3936.22+0×137.724  ×1×1

=3936.22 N

- Longevity of bearing in million rounds:

L=60Lhn106=60×8000×180.02106=86.41 (million revolutions)

- The dynamic load sustainability:

Ctt=QL1m=3936.22×86.4113=17402.03 N <C=27500 N

With: m = 3: Grade of bending line for ball bearing.

=> The condition is satisfied

- Re-calculate the longevity of bearing:

Q3L=C3 ->L=275003936.223=341 (million revolutions)

- Service life of the bearing:

Lh=106L60n=106×34160×180.02=31570.97 (h)

- Test for static load:

Q0=X0×FrB1+Y0×Fa=0.6×3936.22+0.5×137.724=2430.59 (N) and Q0=FrB1=3936.22 (N)

Where:

Q0: Static notational load.

C0: Allowable static notational load..

X0: Coefficient of centripetal load X0=0.6 (based on table 11.6 - Giáo trình Cơ sở thiết kế máy).

Y0: Coefficient of axial load Y0=0.5 (based on table 11.6 - Giáo trình Cơ sở thiết kế máy).

Because Q0=3936.22 N<C0=20200 N  so condition is satisfied.

- Limited number of revolutions of bearing:

Dpwn=const

Where: Dpw: Center diameter of rollers, Dpw=d+D2=50+902=70 (mm)

n: Number of revolutions

Based on Table 11.7 - Giáo trình Cơ sở thiết kế máy, we have:

Dpwn=4.5×105 (mm.rpm) with deep groove ball bearing lubricated with grease.

ngh=4.5×10570=6428.57 (rpm)

Because n<ngh, condition is satisfied.

5.3 Summary

Bearings in shaft I:

| Sign | d, mm | D, mm | B, mm | r, mm | C, kN | C0, kN |
| --- | --- | --- | --- | --- | --- | --- |
| 700109 | 45 | 75 | 10 | 0.5 | 10.5 | 8.57 |

Bearings in shaft II:

| Sign | d, mm | D, mm | B, mm | r, mm | C, kN | C0, kN |
| --- | --- | --- | --- | --- | --- | --- |
| 110 | 50 | 90 | 20 | 2 | 27.5 | 20.2 |

## CHAPTER VI. LUBRICATION METHOD

The contact stress on the gear:

σH=51.281 MPa

The peripheral speed v:

v=4.78 m/s

The hardness of the gear:

HB=255 HB=258 HV

We have the formula:

χbr=10-5×HHV×σH2v=10-5×258×51.28124.78=1.42

Base on the graph 13.9 Cơ sở thiết kế máy - Nguyễn Hữu Lộc

v50=2×10-6 m2/s

Base on the table 13.1, we choose ISO VG 2

# CASE STUDY 2: SCREW JOINT

Exercise 8: A horizontal bar subjected to loads F, N is held tightly by a group of 6 bolts and arranged as shown in Figure 8. Use bolt joints with gaps. Bolts have strength class 3.6. Let's define:

a) Analyze and determine the force acting on each bolt.

b) Diameter d1 and choose bolt.

c) Select bolts in case of using bolt joints without gaps.

Figure 8

Option 7:

- Force F (kN): F = 13 (kN) = 13000 (N)

- L (mm) = 850 (mm)

- a (mm) = 220 (mm)

- b (mm) = 180 (mm)

Note: FM

FF

- Analyze and determine the force acting on each bolt.

First, we specify the center of gravity of the support with 8 bolts. The center of gravity of the bolt group is the intersection 0 of two centerline.

Moving force F to center of gravity 0, we have force F passing through the center of gravity and moment M:

M=F×850×10-3=13000×850×10-3=11050 Nm

Due to the acting of force F, bolts 1-6 the same force FFi:

FFi=F6=130006=2166.667 N

Since the distances from the centers of bolts 1, 2, 4, 6 to the center of gravity 0 are equal, the force caused by the moment M at the bolts has the same value:

FM1=FM2=FM4=FM6=Mr1Σri2=Mr14r12+2r32=11050×102024×102022+2×1102×103

=14957.144 N

Since the distances from the centers of bolts 3, 5 to the center of gravity 0 are equal, the force caused by the moment M at the bolts has the same value:

FM3=FM5=Mr3Σri2=Mr34r12+2r32=11050×1104×102022+2×1102×103

=11576.19 N

Where: r1=r2=r4=r6=22022+18022=10202 mm; r3=r5=110 mm

From the formula of the sum of the force:

Fi=FFi2+FMi2+2×FFi×FMi×Cos(α)

- F1=F6=FF12+FM12+2×FF1×FM1×Cos450

=2166.6672+14957.1442+2×2166.667×14957.144×Cos450

=16560.23 N

- F5=FF52+FM52+2×FF5×FM5×Cos00

=2166.6672+11576.192+2×2166.667×11576.19×Cos00

=13742.86 N

Therefore, the load acting on bolt 1 is the largest: F1=16560.23 N

The tightening force (the required screwing up force) V:

If a bolted joint with a clearance with a factor of safety k = 1.3 and a coefficient of friction f = 0.18 is used, then

V=kF1f=1.3×16560.230.18=119601.66 N

- Determine the diameter d1 and choose the bolt.

The bolts have strength class 3.6 so that σb=300 MPa

- The yield strength σch=0.6×σb=0.6×300=180 MPa

- The allowable tensile stress σk=σchs=1802 = 90 MPa

Where [s] = 2 based on table 17.6 (assume that the diameter of the bolts is from 30÷60 mm)

The diameter d1 of the bolt is	:

d1=4×1.3×Vπ×σk=4×1.3×119601.66π×90=46.9 mm

Select bolt M56 has d1 = 50.046 mm

- Select bolts in case of using bolt joints without gaps.

The bolt diameter due to the use of a gapless bolt joint is determined by the formula:

d0=4×F1π×τ

where τ=0.4σch=0.4×180=72 MPa (Based on table 17.5)

From here:

d0=4×16560.23π×72=17.11 mm

By standard, we select M20,	d1 = 17.249 mm

Exercices 9 Calculate the bolt holding the metal support on the wall as shown in Figure 9.1. The force F acts at an angle  with the horizontal. Static weight (constant). Bolt material has strength grade 5.6. Request:

a) Analyze and determine the force acting on each bolt.

b) Determine the required screwing up force V

c) Diameter d1 and choose the bolt.

Figure 9.1

Option 4:

- Force F (kN): F = 8 (kN) = 8000 (N)

- Angle α (rad) = π/6 (rad)

- a (mm) = 70 (mm)

Figure 9.2

- Analyze and determine the force acting on each bolt.

Hypothesis 1: The bolts are tightened, the joint surface rotates around the axis passing through center of gravity I (Figure 9.2)

1) The force F is classified into two components: FV perpendicular to the joint surface and FH parallel to the joint surface. Moving the force F to the center of gravity I of the joint, we have the component of the bolt axial force:

FV = F×sinπ6=4000 N;

the force lies in the joint surface:

FH = F×cosπ6=6928.2 N

and overturning moment:

M = FH×280+FV×35=2079896 Nmm=2079.9 Nm

- Determine the required screwing up force V.

2) We determine the tightening force V so that the joint surface is not split open:

V = kzFV+MAW(1-χ)

V1 = 1.5104000+2079.9×0.05882.744×10-3(1-0.3)=5099.775 N

We choose k = 1.5 - factor of safety; χ=0.3

z = 10 - number of screws;

FV =4000 N - bolt axial external load; F =8000 N;

A =210×280=58800 mm2=0.0588 m2 - joint surface area;

W = bh26=210×28026=2744000 mm3=2.744×10-3 m3 is the elastic section modulus of the area of the joint

3) Bolt tightening force so that the joint surface does not slip V2

V2 = kFH+f(1-χ)FVfz=1.5×6928.2+0.3×1-0.3×40000.3×10=3744.1 N

4) Equivalent calculated load and bolt diameter

Comparing the two values V1 and V2, we choose the screwing up force (tightening force V1 = 5099.775 N) so that the joint surface is not split open.

- Determine the diameter d1 and choose the bolt.

5) Load acting on the bolt due to the overturning moment component M:

FM1 =FM2=FM3=FM8=FM9=FM10=Mb×Y1ziY12=Mb×Y16Y12+4Y22

FM1 =χ×M×Y16Y12+4Y22=0.3×2079.9×70+702670+7022+47022×10-3=922.1232 N

6) Equivalent calculated load:

Fmax=1.3V+χFVz+FM1

=1.3×5099.775+0.3×400010+922.1232

=7671.831 N

7) Bolt diameter:

The bolts have strength class 5.6 so that σb=500 MPa

- The yield strength σch=0.6×σb=0.6×500=300 MPa

- The allowable tensile stress σk=σchs=3003 = 100 MPa

Where [s] = 3 based on table 17.6 (assume that the diameter of the bolts is from 6÷16 mm)

d1≥4Fmaxπσk=4×7671.831 πσk=9.883 mm

8) By standard, we select bolt M12 with diameter d1 = 10.106 mm

9) Crushing stress in the joint surface:

σcrus=zVA-FVA+MW=10×5099.775210×280-4000210×280+2079896210×28026=1.557 MPa

Therefore, if the crossbar is made of steel, the crushing strength condition is satisfied. Crushing strength is also satisfactory σcrus≤σcrus= 2... 4 MPa even when the crossbar is made of wood or concrete materials.