HO CHI MINH CITY NATIONAL UNIVERSITY

HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY

FACULITY OF MECHANICAL OF ENGINEERING

TRANSMISSION SYSTEM PROJECT

Project 3 – Data no 6

Semester 231

Lecturer: Dr. Lê Thúy Anh

| No | Student | Student ID |
| --- | --- | --- |
| 1 | Huỳnh Nguyễn Toàn Thắng | 2153815 |
| 2 | Đoàn Ngọc Tân | 2153779 |

Ho Chi Minh City, February 21st 2024

# INTRODUCTION

Our nation is progressing along the path of development, with science and technology playing a crucial role in enhancing human life. The increasing application of science and technology is not only boosting labor productivity but also efficiently replacing manual labor, ensuring the safety of workers. Mechanized systems prove to be effective substitutes for human effort in automating production processes, thereby elevating overall labor productivity. By exercising control over these systems, we contribute to the ongoing modernization of automation initiatives in Vietnam.

The Transmission System Project serves as a pivotal component for students specializing in Mechatronics and Mechanical Engineering, providing fundamental insights into the design of mechanical transmission systems. This project enables us to comprehend the intricacies of automated systems in factories, plants, and workshops. The project's scope encompasses the application of knowledge acquired from foundational subjects such as Kinematics and Dynamics of Machines, Machine Elements, and Mechanical Engineering Drawing. Implementation of the project significantly enhances proficiency in AutoCAD and SolidWorks. This newfound skill set, combined with specialized knowledge, allows us to engage with practical systems and develop a comprehensive understanding in preparation for future projects and graduation theses.

We extend our sincere gratitude to Dr. Le Thuy Anh for his dedicated guidance in completing the transmission system project. Additionally, we express our appreciation to the instructors of this semester, as their feedback during project review sessions has been instrumental in expanding our knowledge and refining our drawing skills.

Being our first mechanical system project, some shortcomings in calculations and component selection are inevitable. We look forward to further guidance from our teachers to consolidate our knowledge and gain valuable experiences for our future careers.

TABLE OF CONTENTS

INTRODUCTION	i

CHAPTER 1: MOTOR AND TRANSMISSION RATIO SELECTION	1

1.1	General efficiency of transmission system	1

1.2	Calculate required power	1

1.3	Motor selection	1

1.4	Working power on each shaft	1

1.5	Rotational speed on each shaft	2

1.6	Torque on each shaft	2

1.7 Parameters of transmission system	2

CHAPTER 2: DESIGN BELT CONVEYOR	3

2.1 Select the belt type	3

2.2	Calculate the diameter of the small pulley	3

2.3	Calculate the belt velocity	3

2.4	Calculate the diameter of the big pulley	3

2.5	Calculate the center distance and the belt length L	3

2.6	Check the number of turns in 1 second	3

2.7	Recalculate center distance	4

2.8	Calculate the contact angle	4

2.9	Calculate the number of belts	4

2.10	Width and outer diameter d of pulley	4

2.11 Determine the force on each shaft	5

2.12	Stress in belt drive and service life	5

2.13	Summary	6

CHAPTER 3: DESIGN HELICAL GEAR DRIVE IN SINGLE STAGE REDUCER	7

3.1 Initial parameters	7

3.2	Chose the material	7

3.3	Determine all kinds of the stress on gears:	7

3.4	Calculating geometry parameters of helical gears system:	8

3.5	Peripheral speed v and the angle of helical gear	9

3.6	The value of the forces acting on gear mesh	9

3.7 Dynamic load factors	10

3.8	Checking stress	10

3.9	Tooth shape factor	10

3.10 Check the bending strength	11

3.11	Summary	11

CHAPTER 4: DESIGN TWO SHAFTS IN THE SPEED REDUCER	12

4.1	Choose material	12

4.2	Preliminary selecting diameter of shaft	12

4.3	Distance between bearings and force application point	12

4.4	Calculate the reaction forces acting on shaft I:	13

4.5	Diagram of bending moment and torque diagram of shaft I:	14

4.6	Calculate the reaction forces acting on shaft II:	15

4.7	Diagram of bending moment and torque diagram of shaft II:	16

4.8	Choose the key for the shaft:	17

4.9 Checking for the integral gear shaft condition.	18

4.10	Checking the shaft according to the safety factor	18

CHAPTER 5: CALCULATE AND DESIGN THE BEARING	21

5.1	Calculate the reaction force acting on the bearing in shaft I	21

5.2	Choose accuracy grade for the bearing in shaft I	21

5.3	Calculate the basic dynamic load in shaft I	21

5.4	Test for static load in shaft I	22

5.5	Limited number of revolutions of bearing in shaft I	22

5.6	Calculate the reaction force acting on the bearing in shaft II	22

5.7	Choose accuracy grade for the bearing in shaft II	22

5.8	Calculate the basic dynamic load in shaft II	23

5.9	Test for static load in shaft II	23

5.10	Limited number of revolutions of bearing in shaft II	23

5.11	Summary	24

CHAPTER 6: CALCULATE AND DESIGN THE COUPLING	25

6.1 Initial parameters	25

6.2	Calculate and check the safety factor	25

CHAPTER 7: DESIGNING GEARBOX AND OTHER AUXILIARY ELEMENTS	26

7.1 Initial parameters	26

7.2. Other auxiliary machine elements	27

CHAPTER 8.  THE TOLERANCE	28

8.1. Bearing’s tolerance	28

8.2. Assembly elements	28

8.3.  Summary of assembly tolerance	28

CHAPTER 9. DESIGN OF TRANSMISSION SYSTEM	30

9.1. Extrusion machine	30

9.2. Spiral screw	31

9.3. Design for spiral screw	31

REFERENCES		32

# CHAPTER 1: MOTOR AND TRANSMISSION RATIO SELECTION

## General efficiency of transmission system

According to table 2.3 in reference [1], we select the efficiency of each transmission system parts:

The efficiency of helical gear (x2): ηgear=0.97

The efficiency of V-belt (x1): ηbelt=0.95

The efficiency of sliding-contact bearing (x1): ηsliding=0.98

The efficiency of couplings (x1): ηcouplings=0.98

The efficiency of rolling-contact bearings (x2): ηrolling=0.99

⇒ The general efficiency of transmission system:

ηgeneral=ηbeltηrollingηrollingηgearηcouplingsηslidings

=0.95×0.99×0.99×0.97×0.98×0.98=0.8674

## Calculate required power

Working power of system, based on formula (2.11):

P=Fv1000 EQ/ (Plv,  =1570 ×1.131000=1.77 (kW)

Rotational speed of screw impeller, based on formula (2.16):

n=60000vπD EQ/ (Plv,  =60000×1.13π×230=93.83 (rpm)#

Required power of motor, based on formula (2.8):

Ppre=Pηgeneral EQ/ (Plv,  =1.770.8674=2.04 (kW#)

in which Ppre is the preliminary power for the motor.

## Motor selection

According to the calculated required power, we choose the standard motor power as 2.2 kW. With that standard motor power, we have the following options to choose our motor as well as its rotational speed and transmission ratio.

|  | Rotational speed rpm | General transmission u | V-belt ub | Spur gear ug |
| --- | --- | --- | --- | --- |
| Motor 1 | 2830 | 30.16 | 3.24 | 8 |
| Motor 2 | 1430 | 15.24 | 3.0 | 5 |
| Motor 3 | 945 | 10.07 | 2.52 | 4 |
| Motor 4 | 715 | 7.62 | 1.52 | 3.15 |

Based on table 3.1 Motor SGA (Company CMG, Australia). We choose motor 3 with a rotational speed of 945 rpm (112M).

## Working power on each shaft

Required power of motor:

P=1.77 (kW)

Working power on shaft II:

PII=Pηslidings×ηcoupling=1.770.98×0.98=1.84 (kW)

Working power on shaft I:

PI=PIIηgear×ηrolling=1.840.97×0.99=1.92 kW

Working power of motor:

Pmotor=PIηbelt×ηrolling=1.920.95 × 0.99=2.04 (kW) (satisfied)

## Rotational speed on each shaft

Rotational speed of motor:

nmotor=945 (rpm)

Rotational speed of shaft I:

nI=nmotorubelt=9452.52=375 (rpm#)

Rotational speed of shaft II:

nII=nIugear=3754=93.75 (rpm)

Rotational speed of spiral screw:

nspiral screw=nII  =93.75 (rpm)

## Torque on each shaft

Torque of motor:

Tmotor=9550×Pmotornmotor=9550×2.04945=20.62 (Nm)#

Torque of shaft I:

TI=9550× PInI=9550×1.92375=48.90 (Nm)

Torque of shaft II:

TII=9550×PIInII=9550×1.8493.75=187.43 (Nm)

Torque of spiral screw:

Tspiral screw=9550×Pspiral screwnspiral screw=9550×1.7793.75=180.30 (Nm)

## 1.7 Parameters of transmission system

Table 1-3: Transmission ratio calculation and distribution

| Parameters | Shaft |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Motor | I |  | II |  | Working |
| Power P (kW) | 2.04 | 1.92 |  | 1.84 |  | 1.77 |
| Ratio u | 2.52 |  | 4 |  | 1 |  |
| Rotational speed n (rpm) | 945 | 375 |  | 93.75 |  | 93.75 |
| Torque T (Nm) | 20.62 | 48.90 |  | 187.43 |  | 180.3 |

Where,

Shaft motor: connecting the motor to the V-belt drive.

Shaft I: connecting the V-belt drive to the 1-stage helical spur gear reducer.

Shaft II: connecting the 1-stage helical spur gear reducer to the couplings.

Shaft working: connecting the couplings to the spiral screw.

# CHAPTER 2: DESIGN BELT CONVEYOR

## 2.1 Select the belt type

According to initial parameters (Pmotor=2.2 (kW) and nmotor=945 rpm) and figure 4.1a, page 59 in references [1] we choose belt type B.

According to table 4.5, page 76 references [1], we choose belt type B with:

| Type | A (mm2) | bp (mm) | b0 (mm) | y0 (mm) | h (mm) | d1min (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| B | 138 | 14 | 17 | 4 | 10.5 | 125 |

## Calculate the diameter of the small pulley

Calculate the diameter of the small pulley:

d1=1.2 × dmin=1.2 × 125=150 (mm)#

So, the standard value of d1 is 160 mm.

## Calculate the belt velocity

Velocity of driving pulley:

v1=π×d1×n160000=π × 160× 96060000=7.9168 (m/s)#

In which:

d1: diameter of driving pulley

n1: rotation of driving pulley

If the velocity bigger than 25 m/s, it can lead to lower efficiency, higher temperature, shorten the lifespan of the machine.

The belt velocity is 7.9168 m/s<v=25 (m/s) (satisfied)

## Calculate the diameter of the big pulley

Choose the creep factor: ξ=0.01

Diameter of the big pulley:

d2=d1×u×1-ξ=160×2.52×1-0.01=399.168 (mm)

So, the standard value of d2 is 400 mm.

The real transmission ratio of belt drive:

u'b=d2d1×(1-ξ)=400160×(1-0.01) 2.5253 (mm)#

Error of transmission ratio:

If the error >4%, the machine will not operate as well as we want.

%∆u=|u'b-ub|ub×100%=|2.5253-2.52|2.52=0.21%<4% (satisfied)#

## Calculate the center distance and the belt length L

Calculation for center distance:

2×d1+d2≥a≥0.55×d1+d2+h#

2×160+400≥a≥0.55×160+400+10.5

1120≥a≥318.5

According to table 4.14, page 60 in references [1], we can choose a=480 (mm).

The belt length L:

L=2a+π(d1+d2)2+(d2-d1)24a#

=2 ×480+π×160+4002+400-16024 ×480=1869.626 (mm)

According to the standard value, we choose the belt length L=1800 mm=1.8 m.

## Check the number of turns in 1 second

Check for number of turns:

i=vL=7.916818001000=4.3982 s-1<10 s-1 (satisfied)

## Recalculate center distance

The center distance:

a=k+k2-8∆24#

In which: k and ∆ can be determined by these formulas:

k=L-π×d1+d22=1800-π×160+4002=920.354 (mm)#

∆=d2-d12=400-1602=120 (mm)#

Thus:

a= 920.3541+920.3541 2-8 × 12024=443.959 (mm)

The value of a is still within the acceptable range 1120≥a≥318.5

## Calculate the contact angle

The contact angle:

α1=180-57×d2-d1a=180-57×400-160443.959=149.1864°=2.601 (rad)#

The contact angle needs to bigger than 120° to ensure efficiency, lifespan and avoid reducing transmission capacity as well as friction.

⇔α1>120°

⟹ Satisfy the condition

## Calculate the number of belts

According to the table 4.19, page 62 in references [1], we select:

P0=1.42 kW when L0=2240 mm, d1=160 mm and v1=7.9168 (m/s)

Contact angle correction factor:

Cα=1.24×1-e-α1110=1.24×1-e-149.1864°110=0.9205

Velocity correction factor:

Cv=1-0.05×0.01×v2-1=1-0.05×0.01 × 7.91682-1=0.9772#

Belt length correction factor and allowable effective power:

CL=6LL0=618002240=0.9642#

Transmission ratio correction factor according to Table 4.17 page 61 in reference 1:

Cu=1.14 because u=2.52>2.5

Factor considering the influence of uneven load distribution between belts according to Table 4.18 page 61 in reference 1:

Cz=0.95

Factor considering the influence of the load conditions:

Cr=0.9 (working two shifts, static)

The number of belts:

z≥PIP0CαCuCLCzCrCv=1.921.42×0.9205×1.14×0.9642×0.95×0.9×0.9772=1.6 #

We choose z=2.

## Width and outer diameter d of pulley

According to table 4.21, page 63 in references 1, we have:

t=19 mm; e=12.5 mm; h0=4.2 mm

Width of pulley:

B=z-1t+2e=2-1×19+2×12.5=44 mm#

Outer diameter of smaller pulley:

da1=d1+2h0=160+2×4.2=168.4 mm#

Outer diameter of larger pulley:

da2=d2+2h0=400+2×4.2=408.4 mm#

## 2.11 Determine the force on each shaft

Initial tension force:

F0=A×σ0=z×A1×σ0=2×138×1.2=331.2 (N)

Tension force on each belt:

F0z=331.2 2=165.6 (N)#

Circumferential force:

Ft=1000×P1v1=1000×1.927.9168=242.52 (N)#

Circumferential force on each belt:

Ftz=242.522=121.26 (N)

Shaft load:

Fr≈3×F0×sin α12=3×331.2×sin2.6010 rad2=957.52 (N)#

Force on the tight side:

F1=Fo+Ft2=331.2+242.522=452.46 (N)#

Force on the loose (slack) side:

F2=Fo-Ft2=331.2-242.522=209.93 (N)

Centrifugal tension according to formula page 64 in reference1:

Based on the type B belt drive, we have qm=0.178 kg/m

Fv=qm×v2=0.178×7.91682=11.15 (N)#

in which qm is the length density of belt (kg/m)

## Stress in belt drive and service life

Maximum belt stress in belt drive:

σmax=F0A+Ft2A+ρmv2×10-6+2×y0d1×E#

=331.22×138+242.522×138+1100×7.91682×10-6+2×4×100160

⇒ σmax=6.7 MPa<10 MPa=σk (satisfied)

If maximum belt stress exceeds the threshold of 10 MPa, the belt can become damaged and torn, shaft also can be misaligned.

Service life:

Lh=σrσmaxm×1072×3600×i=96.78×1072×3600×4.3982=3347.6048 (hours)#

In which:

Fatigue strength of belt σr=9 MPa (V-belt)

Exponent of fatigue curve m=8 (V-belt)

## Summary

Table 2-1: Parameter for V-belt drivers

| Parameter | Value |
| --- | --- |
| Belt type | B-type V-belt |
| Number of the belts z | 2 |
| Center distance a (mm) | 443.959 |
| Belt length L (mm) | 1800 |
| Warp angle α1 (°) | 149.1864 |
| Pulley’s width B (mm) | 44 |
| The number of Belt's turn per second (s-1) | 4.3982 |
| Driving pulley’s diameter da1 (mm) | 160 |
| Driven pulley’s diameter da2 (mm) | 400 |
| Maximum belt stress | 6.7 |
| Initial tension F0 (N) | 331.2 |
| Tangential force Ft(N) | 242.52 |
| Shaft load Fr (N) | 957.52 |
| The force on the tight side | 452.46 |
| The force on the loose (slack) side | 209.93 |
| Service life of belt Lh (hours) | 3347.6048 |

# CHAPTER 3: DESIGN HELICAL GEAR DRIVE IN SINGLE STAGE REDUCER

## 3.1 Initial parameters

| Name | Value |
| --- | --- |
| Power transmitting, PI kW | 1.92 |
| Rotational speed, nI rpm | 375 |
| Torque, TI Nmm | 48.90 |
| Transmission ratio, ugear | 4 |

## Chose the material

According to table 6.1 in reference [1], we choose:

Driving gear: choose C45 steel with hardness HB1=245 HB, through induction hardening, structural improvement, σb1=850 MPa,σch1=580 MPa.

To increase teeth wear, the hardness of driven gear needs to follow the formula:

H1≥H210-15HB

Driven gear: choose C45 steel with hardness HB2=230 HB, through structural improvement, σb2=750 MPa,σch2=450 MPa.

## Determine all kinds of the stress on gears:

According to table 6.2*, we have:

Limitation of contact stress σHlimo :

σHlim10=2HB1+70=2×245+70=560 MPa

σHlim20=2HB2+70=2×230+70=530 (MPa)

Limitation off bending stress σFlimo :

σFlim10=1.8HB1=1.8×245=441 MPa

σFlim20=1.8HB2=1.8×230=414 MPa

Safety coefficient of contact stress: SH=1.1

Safety coefficient of bending stress: SF=1.75

Coefficient considering the impact of loading:

KFC=1 (𝑜𝑛𝑒 − 𝑤𝑎𝑦 𝑡𝑟𝑎𝑛𝑠𝑚𝑖𝑠𝑠𝑡𝑖𝑜𝑛)

Number of cycles of change in contact stress NHO :

NHO1=30 HB12.4=30×2452.4=16259974.39≈16.259974 ×106

NHO2=30 HB22.4=30×2302.4=13972305.13≈13.972305 ×106

Number of cycles of change in bending stress:

NFO1 =NFO2=4×106

Working time:

Lh=La×365×Kyear×24×Kn=7×300×2×8=33600 (hours)

The equivalent number of cycles if the gear is working with working load and number of revolution n is constant NHE=NFE:

NHE1=NFE1=60×c×n1×Lh

=60×1×375×33600=75.6×107

NHE2=NFE2=60×c×n2×Lh

=60×1×93.75×33600=18.9×107

Because NHE1>NHO1; NHE2>NHO2; NFE1>NFO1; NFE2>NFO2

⇒KHL1=KHL1=KFL1=KFL2=1

Allowable contact stress and bending stress:

σH1=0.9× σHlim10×KHL1 sH1=0.9×560×1 1.1=458.18 MPa

σH2=0.9× σHlim20×KHL2 sH2=0.9×530×1 1.1=433.64 MPa

σF1=σFlim1oKFL1SF1=441×11.75=252 MPa

σF2=σFlim2oKFL2SF2=414×11.75=238.57 MPa

Because of helical gear, we calculate allowable stress by the equation below:

[σH]=0.5×σH12+σH22=0.5×458.182+433.642=446.08 MPa

Checking with the condition: [σH]min≤[σH]≤1.25[σH]min (satisfied)

Allowable contact stress when overloading:

σHmax=2.8σch1=2.8×580=1624 (MPa)

Allowable bending stress when overloading:
σFmax1=0.8σch1=0.8×580=464 (MPa)

σFmax2=0.8σch2=0.8×450=360 (MPa)

## Calculating geometry parameters of helical gears system:

According to table 6.6*, we have coefficient of gear width:

ψba=0.3

From here infer:

ψbd=ψba u+12=0.34+12=0.75

According to table 6.7*, we have coefficient of unequal load distribution on the gear:

KHβ=1.03; KFβ=1.07

Calculation for the center distance of both gears:

aw=430u+13T1KHβψba[σH]2u=430×4+1348.9×1.030.3×446.082×4

=127.5 mm

We choose the standard center distance, aw1=150mm (𝑆𝑡𝑎𝑛𝑑𝑎𝑟𝑑 𝑆𝐸𝑉229 − 75).

The module m can be calculated according to the center distance:

m=0.0125÷0.025×aw=0.01÷0.02×150=1.5÷3#

We choose m=3

The condition of the tooth angle: 8°≤β≤20°

Based on equation (6.31)*:

cos8o≥mnz1u+12aw≥cos20°#

2awcos8omu+1≥z1≥2awcos20omu+1

2×150×cos8o3×4+1≥z1≥2×150×cos20o3×4+1

19.8054≥z1≥18.7939

We choose the number of teeth on the driving gear: z1=19

The number of teeth on the driven gear: z2=u1z1=4× 19=76

Lead angle of helical gear β:

β=arccosmn(z1+z2)2aw=arccos3×19+762×150=18.1949°

Pitch circle diameter of the driving gear:

d1=z1mcosβ=19×3cos18.1949°=60 (mm)#

Pitch circle diameter of the driven gear:

d2=z2mcosβ=76×3cos18.1949°=240 (mm)

Rolling circle diameter:

dw1=d1=60 (mm)

dw2=d2=240 (mm)

Outside diameter of driving gear:

da1=d1+2m=60+2×3=66 (mm)#

Outside diameter of driven gear:

da2=d2+2m=240+2×3=246 (mm)

Root diameter of driving gear:

df1=d1-2.5×m=60-2.5×3=52.5 (mm)#

Root diameter of driven gear:

df2=d2-2.5×m=240-2.5×3=232.5 (mm)

Driven gear width:

b2=aw×ψba=150×0.3=45=bw (mm)#

Driving gear width:

b1=b2+5=45+5=50 (mm)

Pressure angle:

αnw=20°

Pressure angle in helical gear:

tanαtw=tanαnwcosβ

⇒αtw=20.9632°

## Peripheral speed v and the angle of helical gear

v=πd1n160000=π×60×37560000=1.18 (m/s)

According to table 6.13, page 106 in reference [1], we choose the grade of accuracy is 9

## The value of the forces acting on gear mesh

Peripheral force Ft:

Ft1=Ft2=2TI×103d1=2×48.90×10360=1630 N#

Axial force Fa:

Fa1=Fa2=Ft1tanβ=1630×tan18.1949=535.76 N#

Radial force Fr:

Fr1=Fr2=Ft1tanαcosβ=1630×tan20°cos18.1949=623.5 N

The normal force Fn:

Fn1=Fn2=Ft1cosβcosα=1630cos18.1949×cos20=1825.91 N#

Figure 3.2 Forces acting on gears

## 3.7 Dynamic load factors

Based on the table 6.14, we choose level of accuracy is 9, so we have KHα=1.13; KHα=1.37

Calculating the factor vH:

vH=δHg0vaw1u1=0.002×73×1.18×1504=1.05 (m/s)

The factor of kinematic load appears on meshing zone:

KHv=1+vhbw1dw12T1KHβKHα=1+1.05×45×602×4890×1.03×1.37=1.21

KFv=1+vhbw1dw12T1KFβKFα=1+1.05×45×602×4890×1.07×1.13=1.24

We can calculate the total load factor when contacting and bending:

KH=KHβKHαKHv=1.03×1.37×1.21=1.14

KF=KFβKFαKFv=1.07×1.13×1.24=1.5

## Checking stress

σH=ZMZHZεFt1KHu+1dw1bwu ≤σH

Where:

ZH=2cosβbsin2αtw=2×cos18.1949sin2×20=1.72

Zε=1εα=0.7754

According to formula 6.38b, page 105 in reference [1], we have:

εa=1.88-3.21z1+1z2cosβ

=1.88-3.2119+176cos⁡(18.1949)=1.5860≥1

According to table 6.5, page 96 in reference [1], we have:

ZM=274 MPa13

According to formula 6.39, page 106 in reference [1], we have:

KH=KHβ×KHv×KHα=1.03×1.02×1.13=1.1871#

σH=ZMZHZε2T1KHu+1d12b1u

=274×1.72×0.7754×2×48.90×103×1.4×4+1602×55×4=339.75 (MPa)

As σH<σH (339.75<458.18), the gear has enough contact strength.

## Tooth shape factor

Equivalent gear teeth:

zv1=z1cos3β=19cos318.1949=22 teeth#

zv2=z2cos3β=76cos318.1949=88 teeth#

Tooth shape factor:

YF1=3.47+13.2zv1=3.47+13.222=4.07

YF2=3.47+13.2zv2=3.47+13.288=3.62

## 3.10 Check the bending strength

σF=YFFtKFYεYβbwmn#

When:

KF=KFαKFβKFv=1.37×1.07×1.04=1.5245#

YF1=4.07

YF2=3.62

Yε=1εa=11.6632=0.6013#

Yβ=1-β120=1-18.1949120=0.8483#

Ft=2T1d1=2×48.90×10380=1222.5 N#

σF1YF1=2524.07=61.9165; σF2YF2=238.573.62=65.9033 #

Because σF1YF1<σF2YF2 so we choose driving gear to calculate:

σF1=YF1FtKFYεYβb1mn=4.07×1630×1.5×0.6013×0.848355×3#

=30.76 MPa<σF1max=464 (MPa)

As σF1<σF1, the gear has enough bending strength.

## Summary

Table 3.1 Helical gear final specification

| Parameter | Sign | Driving wheel | Driven wheel |
| --- | --- | --- | --- |
| Center distance (mm) | aw | 150 |  |
| Module | mn | 3 |  |
| Number of teeth (teeth) | z | z1=19 | z2=76 |
| Peripheral speed (m/s) | v | 1.18 |  |
| The pitch diameter (mm) | d | d1=60 | d2=240 |
| The pitch outside diameter (mm) | da | da1=66 | da2=246 |
| The root diameter (mm) | df | df1=52.5 | df2=232.5 |
| Lead angle of helical gear | β | 18.1949° |  |
| Peripheral force (N) | Ft | Ft1=Ft2=1630 |  |
| Radial force (N) | Fr | Fr1=Fr2=623.5 |  |
| Axial force (N) | Fa | Fa1=Fa2=535.76 |  |
| Normal force (N) | Fn | Fn1=Fn2=1825.91 |  |

# CHAPTER 4: DESIGN TWO SHAFTS IN THE SPEED REDUCER

## Choose material

Given that we are in the process of designing a gearbox, it is advisable to opt for C45 steel, characterized as a medium carbon steel. The selected heat treatment method is soft hardening. The benefit of selecting this material lies in its cost-effectiveness and the wide range of applications it offers.

According to table 6.1 in references [1], we choose:

Material: C45 Steel

Method: Normalization, structural improvement

Hardness: H=245 HB

Ultimate strength: σb=850 MPa

Yield limit: σch=580 Mpa

## Preliminary selecting diameter of shaft

Preliminary choosing allowable torsional stress τ=15 MPa for shaft I and τ=20 MPa for shaft II because shaft ΙΙ suffer from bigger torsion than shaft Ι.

Preliminary diameter of shaft:

Shaft I:

d1≥3T10.2τ=348.90×1030.2×15=25.35 mm#

⇒ According to standard, we select preliminary diameter of shaft I: d1=30 mm

Shaft II:

d2≥3T20.2τ=3187.43×1030.2×20=33.47 mm#

⇒ According to standard, we select preliminary diameter of shaft II: d2=35 mm

## Distance between bearings and force application point

Shaft I:

According to table 10.2 in reference [1], we choose the width of bearing: bo1=19 mm

According to table 10.3 in reference [1], we choose:

k11=10 (mm): Distance from driving gear’s face to gearbox inner surface or between rotating elements.

k12=10  (mm): Distance from bearing face to inner gearbox-wall.

k13=15 (mm): Distance from element face to gearbox cover.

h1n=15 (mm): Length from bearing cover to bolt.

Mayo length of helical spur gear:

lm12=1.5d1=45 (mm)

Mayo length of belt drive:

lm13=1.3÷1.5(d1+5)=42÷52.5 (mm)

Because b1=50 mm we choose lm12=50 mm so it has the length as the gear’s width.

Distance between elements on shaft:

l13=0.5lm13+bo1+k11+k12=0.550+19+10+10=54.5 mm#

l11=2l13=2×54.5=109 mm#

l12=0.5lm12+bo1+k13+h1n=0.545+19+15+15=62 mm#

Shaft II:

According to table 10.2 in reference [1], we choose the width of bearing: bo2=21 mm

According to table 10.3 in reference [1], we have:

k21=7.5 mm: Distance from driving gear’s face to gearbox inner surface or between rotating elements

k22=5 mm: Distance from bearing face to inner gearbox-wall.

k23=15 mm: Distance from element face to gearbox cover.

h2n=15 mm: Length from bearing cover to bolt.

Mayo length of screw drive:

lm21=(1.2÷1.5)d2=(1.2÷1.5)35=(42÷52.5) mm#

We chose lm21=45 (mm)

Mayo length of belt drive:

lm23=1.2÷1.5(d2+1)=1.2÷1.540=48÷60 mm#

Because b2=45mm we choose lm23=b2=45 mm so it has the length as the gear’s width Distance between elements on shaft:

l21=0.5×lm23+bo2+k21+k22=0.5×45+21+7.5+5=45.5 mm#

We choose l21=54.5 (mm)

l23=0.5lm21+bo2+k23+h22=0.545+21+15+15=63 mm#

l22=2l21=2×54.5=109 mm#

## Calculate the reaction forces acting on shaft I:

Force acting from helical gear:

Peripheral force: Ft1=1630 (N)

Centrifugal force: Fr1=623.5 (N)

Axial force: Fa1=535.76 (N)

Moment caused by axial force:

Ma1=Fa1×d12=535.76×60×10-32=16.07 (Nm)#

The torque acting on the shaft:

TI=48.9 (Nm)

According to free body diagram in figure, we have force equation of equilibrium and moment equation of equilibrium in Oyz  plane:

∑MD=FBy×109×10-3+Fr1×54.5×10-3-Fvb×166×10-3-Ma1=0

∑FOy=FDy+Fvb-FBy-Fr1=0

⟺FBy=640.95 (N)

⇒FDy=735.69 (N)

According to free body diagram in figure, we have force equation of equilibrium and moment equation of equilibrium in Oxz plane:

∑MD=FBx×109×10-3-FHb×166×10-3+Ft1×54.5×10-3=0

∑FOx=FDx-FHb+FBx-Ft1=0

⟺FBx=1318.18 (N)

⇒FDx=642.22 (N)

## Diagram of bending moment and torque diagram of shaft I:

Figure 4.1 Bending moment and torque diagram of shaft I

According to moment distribution diagram, the most critical position is section C:

MtdC=MCx2+MCy2+0.75×T12#

=51.342+31.322+0.75×48.92=73.55 (Nm)

Equivalent moment at A, B, D:

MtdA=MAx2+MAy2+0.75×T12#4.21

=02+02+0.75×48.92=42.35 Nm

MtdB=MBx2+MBy2+0.75×T12

=18.832+30.142+0.75×48.92=36.05 (Nm)

MtdD=MDx2+MDy2+0.75×T12#

=02+02+0.75×02=0 (Nm)

According to table 10.5 in reference [1] with d1=30 mm, we choose allowable bending stress [σ]=67 MPa.

Shaft diameters are determined:

dB≥3MtdB0.1×σ=336.05×1030.1×67=17.52 mm#

According to standard, we choose dB=30 mm.

dD≥3MtdD0.1×σ=30×1030.1×67=0 mm#

We choose dD=30 mm as it is the other bearing of this shaft, so they have to be the same size.

dA≥3MtdA0.1×σ=342.35×1030.1×67=18.49 mm

Because at position A there is a keyway, according to the standard we can choose dA=20 mm.

dC≥3MtdC0.1×σ=373.55×1030.1×67=22.22 mm#

Because at position C there is a keyway, according to the standard we can choose dC=40 mm.

## Calculate the reaction forces acting on shaft II:

Force acting on shaft II:

Force acting from helical gear:

Tangential force: Ft2=1630 N

Centrifugal force: Fr2=623.5 N

Axial force: Fa2=535.76 N

Force acting on coupling:

Fc=2T2Do=2×187.43230×10-3=1629.83 (Nm)

The radial force acting on the shaft from the coupling:

Frc=0.2...0.3×Fc=0.2...0.3×1629.83=325.966…488.949 (N)

We choose Frc=400 (N)

Moment caused by axial force:

Ma2=Fa2×d222=535.76×240×10-32=64.29 (Nm)

The torque acting on the shaft:

TII=187.43 (Nm)

According to free body diagram in figure, we have force equation of equilibrium and moment equation of equilibrium in Oyz plane:

∑MB=FDy×109×10-3+Fr2×54.5×10-3-Ma2=0

∑FOy=FBy+FDy-Fr2=0#

⟺FDy=278.08 (N)

⇒FBy=345.42 (N)

According to free body diagram in figure, we have force equation of equilibrium and moment equation of equilibrium in Oxz plane:

∑MB=FDx×109×10-3-Frc×117.5×10-3-Ft2×54.5×10-3=0#

∑FOx=FDx-Ft2-Fc+FBx=0

⟺FDx=1446.19 (N)

⇒FBx=583.81 (N)

## Diagram of bending moment and torque diagram of shaft II:

Figure 4.2 Bending moment and torque diagram of shaft II

According to moment distribution diagram, the most critical position is section C:

MtdC=MCx2+MCy2+0.75×T22#

=31.822+49.142+0.75×187.432=172.55 (Nm)

Equivalent moment at A, B, D:

MtdA=MAx2+MAy2+0.75×T22

=02+02+0.75×187.432=151.32 Nm

MtdB=MBx2+MBy2+0.75×T22#

=02+02+0.75×02=0 (Nm)

MtdD=MDx2+MDy2+0.75×T22#

=25.2 2+02+0.75×187.432=164.26 (Nm)

According to table 10.5 in reference [1] with d2=35  mm, we choose allowable bending stress [σ]=65 MPa.

Shaft diameters are determined:

dD≥3MtdD0.1×σ=3164.26 ×1030.1×65=29.35 mm

According to standard, we choose dD=40 mm.

dB≥3MtdB0.1×σ=30×1030.1×65=0 mm#

We choose dB=40 mm as it is the other bearing of this shaft, so they have to be the same size.

dC≥3MtdC0.1×σ=3172.55×1030.1×65=29.83 mm#

Because at position C there is a keyway, according to the standard we can choose dC=50 mm.

dA≥3MtdA0.1×σ=3151.32×1030.1×65=28.55 mm#

Because at position A there is a keyway, according to the standard we can choose dA=32 mm.

## Choose the key for the shaft:

We choose the same material with the gear C45, heat treatment is normalization, structural improvement.

According to table 9.1a in reference [1], we choose key length lt following the standard lt=

0.8…0.9lm, choose key parameter b×h following largest cross-section on shaft.

Check the condition for the volume stress:

σd=2Tdlt(h-t1)≤σd

τc=2Tdblt≤τc

With:

σd: Volume stress

τc: Shear stress

𝑇: Torque on shaft at special position

𝑑: Diameter of the special position

b, lt, ℎ, t1 : Parameter of the key

[σd]=150 MPa: Allowable volume stress (according to table 9.5 in reference [1], the key is C45 and the system is static load)

τc=60÷90 MPa: Allowable shear stress

Table 4.1 Selection of key dimensions

| Shaft | Cross-section | d (mm) | lt (mm) | b (mm) | h (mm) | t1 mm | T (Nm) | σd (MPa) | τc (MPa) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | A | 20 | 40 | 6 | 6 | 3.5 | 48.9 | 48.9 | 20.38 |
|  | C | 40 | 40 | 12 | 8 | 5 | 48.9 | 20.4 | 5.09 |
| II | A | 30 | 36 | 10 | 8 | 5 | 187.43 | 115.7 | 34.71 |
|  | C | 30 | 40 | 10 | 8 | 5 | 187.43 | 71.4 | 17.85 |

⇒ All the feather keys are satisfied.

## 4.9 Checking for the integral gear shaft condition.

X≤2.5×m

With:

X: The distance between the root diameter and the top of the keyway.

m: The module of the gear.

Checking for the shaft I:

X=df1-dC2-t2=52.5-402-3.3=2.95 mm≤2.5×m=7.5 mm#

The shaft I is the driving shaft with the integral gear, so we do not need the key at the gear position.

Checking for the shaft II:

At the gear in the reducer:

X=df2-d2-t2=232.5-502-3.3=87.95 mm≥2.5×m=7.5 mm#

At the external gear:

X=df2-d22-t2=232.5-352-3.3=95.45 mm≥2.5×m=7.5 mm#

The shaft II is the shaft without the integral gear.

## Checking the shaft according to the safety factor

The structure of the shaft has been designed to ensure durability if the safety factor at critical sections meets the following conditions:

s=sσsτsσ2+sτ2≥s #

With:

[s]=2.5÷3: Allowable safety factor

sσ, sτ: Safety factor is only considered for bending and torsional stresses, determined by the following the formula:

sσ=σ-1Kσσaεσβ+ѱσσm#

sr=τ-1Kττaετβ+ѱττm#

Where:

σ-1: Ultimate bending stress corresponding to each symmetrical cycle for carbon steel

σ-1=0.436×σb=0.436×850=370.6 MPa

τ-1: Ultimate torsional stress corresponding to each symmetrical cycle for carbon steel

τ-1≈0.58×σ-1=0.58×370.6=214.9 MPa

Kσ,Kτ: According to table 10.12 in reference [1], we have:

Kσ=2.05; Kτ=1.9

ѱσ,ѱτ: According to table 10.7 in reference [1], we have:

ѱσ=0.1; ѱτ=0.05

εσ, ετ: According to table 10.10 in reference [1], we have:

εσ-I=0.88; ετ-I=0.81; εσ-II=0.85; ετ-II=0.78

β: Surface fatigue strength factor. According to table 10.9 in reference [1], we choose: β=1.7

σm, σa: Due to the rotation shaft, the bending stress changes according to the symmetric cycle:

σm=0; σa=σmax=MW#

τm, τa: When the shaft rotates in one direction, the torsional stress changes according to the cycle:

τa=τm= τmax2=T2Wo#

With:

M=Mx2+My2#

W=πd332-bt1d-t122d#

Wo=πd316-bt1d-t122d#

Table 4.2 Parameters for strength check

| Shaft | Cross-section | Key b×h×t1 | W(mm3) | Wo(mm3) | εσ | ετ | σa | τa | sσ | sτ | s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | A (20) | 6×6×3.5 | 642.47 | 1428.86 | 0.88 | 0.81 | 0 | 19 | - | 7.6 | - |
|  | B (30) | - | 2650.72 | 5301.43 | 0.88 | 0.81 | 13.4 | 4.6 | 20.2 | 31.6 | 17 |
|  | C (40) | 12×8×5 | 6114.44 | 11647.62 | 0.88 | 0.81 | 10.9 | 2.1 | 24.8 | 69.2 | 23 |
|  | D (30) | - | 2650.72 | 5301.43 | 0.88 | 0.81 | 0 | 4.6 | - | 31.6 | - |
| II | A (32) | 10×8×5 | 2025.72 | 4780.6 | 0.85 | 0.78 | 0 | 19.6 | - | 7.2 | - |
|  | B (40) | - | 6283.19 | 12566.37 | 0.85 | 0.78 | 0 | 7.5 | - | 18.7 | - |
|  | C (50) | 14×9×7.5 | 10375.28 | 23189 | 0.85 | 0.78 | 5.6 | 4 | 46.4 | 25 | 22 |
|  | D (40) | - | 6283.19 | 12566.37 | 0.85 | 0.78 | 4 | 7.5 | 65.3 | 18.7 | 18 |

⇒ The shafts satisfy fatigue conditions.

Test for static durability

To prevent large elasticity deformation on shafts or break down when being subjected to sudden load (such as when starting the motor). Testing each shaft static durability at the section where the largest moment is located, the result must satisfy the equation below:

σtđ=σ2+3τ2≤σqt≈0.8×σch=0.8×580=464 MPa#

Where:

σ=Mmax0.1×d3#

τ=Tmax0.2×d3#

Shaft I:

The most critical position is section C:

σC=Mmax0.1×dC3=67.12×1030.1×403=10.5 MPa#

τD=Tmax0.2×dC3=48.9×1030.2×403=3.8 MPa#

σtđ=σ2+3τ2=10.52+3×3.82=12.4 MPa#

⇒σtđ≤σqt=464 (MPa)

⇒ The shaft satisfies the static durability test.

Shaft II:

The most critical position is section B:

σB=Mmax0.1×dD3=58.54×1030.1×403=9.1 MPa#

τB=Tmax0.2×dD3=187.43×1030.2×403=14.6 MPa#

σtđ=σ2+3τ2=9.12+3×14.62=26.9 MPa#

⇒σtđ≤σqt=464 (MPa)

⇒ The shaft satisfies the static durability test.

# CHAPTER 5: CALCULATE AND DESIGN THE BEARING

## Calculate the reaction force acting on the bearing in shaft I

Calculate the reaction force acting on the bearing in shaft I

Centrifugal force at B:

FrB=FB=FBx2+FBy2=1318.182 +640.982=1465.76 N#

Centrifugal force at D:

FrD=FD=FDx2+FDy2=642.222+735.712=976.58 N#

Due to FrB>FrD so we choose FrB

Fa1FrD=535.761465.76=0.366#

This satisfies the condition that Angular contact ball bearing is suggested:

FaFr>0.3

⇒ Select Angular contact ball bearing.

According to Table P2.12* in reference [1] we have information about the bearing:

Table 5.1 Parameter of bearing on shaft I

| ID | d (mm) | D (mm) | B (mm) | r (mm) | C (kN) | Co (kN) |
| --- | --- | --- | --- | --- | --- | --- |
| 46306 | 30 | 72 | 19 | 2 | 25.6 | 18.17 |

## Choose accuracy grade for the bearing in shaft I

Choose grade 0 as it both is cheap and suitable with the system.

## Calculate the basic dynamic load in shaft I

Fa1C0=535.7618170=0.029

⇒e=0.34

According to table 11.4 in reference [1] with:
Fa1VFrD=0.366>e=0.34

⇒ X=0.45; Y=1.62

In which:

V: Coefficient of which ring rotate V=1

We calculate the load at A with the formula:

Q=Qr=XVFr+YFaKtKđ

In which:

Kđ: Coefficient of load specificity Kđ=1 (based on table 11.3 in reference [1] for static load)

Kt: Coefficient of temperature Kt=1

Q=Qr=XVFr+YFaKtKσ#

=0.45×1×1465.76+1.62×535.76 ×1×1=1527.52 (N)

Longevity of bearing in million rounds:

L=60Lhn1106=60×33600×375106=756 million revolutions#

The dynamic load sustainability:

Ctt=QL1m=1527.52×75613=13915.35 N<C=25600 N#

In which:

m=3: Grade of bending line for ball bearing

⇒ The condition is satisfied.

Re-calculate the longevity of bearing:

Q3L=C3⇒L=4707 million revolutions#

Service life of the bearing:

Lh=106L60n=106×4707 60×375=209200 h#

## Test for static load in shaft I

Q0=X0×Fr+Y0×Fa=0.5×1465.76+0.47×535.76=984.69 N#

Q0<FrD=1465.76 (N)

⇒Choose Q0=1465.76 (N)

Where:

Q0: Static notational load.

C0: Allowable static notational load.

X0: Coefficient of centripetal load X0=0.5 (based on table 11.6 in reference [1])

Y0: Coefficient of axial load Y0=0.47 (based on table 11.6 in reference [1])

Because Q0=1456.76N<C0=18170 (N), condition is satisfied.

## Limited number of revolutions of bearing in shaft I

Limited number of revolutions of bearing:

Dpwn=const#

Where: Dpw: Center diameter of rollers:

Dpw=d+D2=30+722=56 mm#

n: Number of revolutions

Based on Table 11.7 in reference [1] we have:

Dpwn=3.5×105 mm.rpm#

With angular contact bearing lubricated by grease:

ngh=3.5×10530=11666.66 rpm

Because n<ngh, condition is satisfied.

## Calculate the reaction force acting on the bearing in shaft II

Centrifugal force at A:

FrD=FD=FDx2+FDy2=1446.192 +278.082=1472.68 N#

Centrifugal force at B:

FrB=FB=FBx2+FBy2=583.812+345.422=678.34 N#

Due to FrD>FrB so we choose FrD

Fa2FrD=535.76 1472.68=0.36#

This satisfies the condition that deep grove ball bearing is suggested:
FaFr>0.3

⇒ Select deep grove ball bearing.

According to Table P2.7* in reference [1] we have:

Table 5.2 Parameter of bearing on shaft II

| Sign | d (mm) | D (mm) | B (mm) | r (mm) | C (kN) | Co (kN) |
| --- | --- | --- | --- | --- | --- | --- |
| 46308 | 40 | 90 | 23 | 2.5 | 39.2 | 30.7 |

## Choose accuracy grade for the bearing in shaft II

Choose grade 0 as it both is cheap and suitable with the system.

## Calculate the basic dynamic load in shaft II

Fa2C0=535.7630700=0.017

⇒e=0.19

According to table 11.4 in reference [1] with

Fa2VFrD=0.36>e=0.19#

⇒ X=0.56; Y=2.3

In which:

V: Coefficient of which ring rotate V=1

We calculate the load at A with the formula:

Q=Qr=XVFr+YFaKtKσ#

In which:

Kσ: Coefficient of load specificity Kσ=1 (based on table 11.3 in reference [1] for static load)

Kt: Coefficient of temperature Kt=1

Q=Qr=XVFr+YFaKtKσ#

=0.56×1×1472.68+2.3×535.76×1×1=2056.95 (N)

Longevity of bearing in million rounds:

L=60Lhn2106=60×33600×187.43106=377 million revolutions#

The dynamic load sustainability:

Ctt=QL1m=2056.95×37713=14859 N<C=39200N#

In which:

m=3: Grade of bending line for ball bearing

⇒ The condition is satisfied.

Re-calculate the longevity of bearing:

Q3L=C3⇒L=6921 million revolutions

Service life of the bearing:

Lh=106L60n=106×6921 60×187.43=615429 h

## Test for static load in shaft II

Q0=X0×Fr+Y0×Fa=0.6×2173.99+0.5×535.75=1572.27 N#

Because Q0>FrA=1472.68 (N)

⇒Choose Q0=1572.27 (N)

Where:

Q0: Static notational load.

C0: Allowable static notational load.

X0: Coefficient of centripetal load X0=0.5 (based on table 9.6 in reference [1])

Y0: Coefficient of axial load Y0=0.47 (based on table 9.6 in reference [1])

Because Q0=1572.27 N<C0=30700 (N), condition is satisfied.

## Limited number of revolutions of bearing in shaft II

Limited number of revolutions of bearing:

Dpwn=const#

Where: Dpw: Center diameter of rollers:

Dpw=d+D2=40+902=65 mm#

n: Number of revolutions

Based on Table 11.7 in reference [1] we have:

Dpwn=3.5×105 mm.rpm

With angular contact bearing lubricated by grease.

ngh=3.5×10540=8750 rpm#

Because n<ngh, condition is satisfied.

## Summary

Table 5.3 Bearings final specification

| Shaft | Sign | d (mm) | D (mm) | B (mm) | r (mm) | C (kN) | Co (kN) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I | 46306 | 30 | 72 | 19 | 2 | 25.6 | 18.17 |
| II | 46308 | 40 | 90 | 23 | 2.5 | 39.2 | 30.7 |

# CHAPTER 6: CALCULATE AND DESIGN THE COUPLING

## 6.1 Initial parameters

Torque acting on shaft I: T=187.43 Nm

The rotational speed of the chain coupling: n=93.75 rpm

To decrease the impact of shaft, we select elastic coupling.

We have information about elastic coupling based on table 16-10a*:

Table 6.1 Coupling specification

| [T] (Nm) | nmax (rpm) | d (mm) | dm (mm) | l (mm) | D (mm) | D0 (mm) | L (mm) | d1 (mm) | Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 250 | 3800 | 32 | 65 | 110 | 140 | 105 | 165 | 56 | 6 |

Table 6.2 Dimensions of the elastic ring

| [T] (Nm) | dc (rpm) | d1 (mm) | D2 (mm) | l (mm) | l1 (mm) | l2 (mm) | l3 (mm) | h (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 250 | 14 | M10 | 20 | 62 | 34 | 15 | 28 | 1.5 |

According to table 16-1*, we select the coefficient of working mode:

k=1.2 (the machine is belt conveyor)

We can calculate the moment torque of coupling:

Tt=kT=1.2×187.43=224.9<T (satisfied)

## Calculate and check the safety factor

σd=2kTZD0dcl3=2×1.2×187.43×1036×105×14×28=1.82 (MPa)

σu=kTl00.1dc3D0Z=1.2×187.43×103×620.1×6×143×105=53.99 (MPa)

⇒σd≤σd=(2÷4) MPa(satisfied)

⇒σu≤σu=(60÷80) MPa(satisfied)

# CHAPTER 7: DESIGNING GEARBOX AND OTHER AUXILIARY ELEMENTS

7.1 Initial parameters

The center distance between shafts: a=150 mm

The length of shaft I and shaft II: l1=187 mm and l2=200 mm

    - Choose the material for gearbox cover

The gear housing is crucial in gearbox design, ensuring the machine's components stay secure, distributing loads evenly, and safeguarding against dust.

The critical standard for the gearbox is to have high stiffness and low weight, so cast iron GX15-32 is used as material. To facilitate installation, cap and housing unit are cast separately.

    - Calculation and designing gearbox cover

Table 7.1 Table of parameters and calculation of gearbox cover

|  | Parameters | Calculation |
| --- | --- | --- |
| Width | Gearbox body, δ (mm) | δ=0.03a+3≈7.5 |
|  | Gearbox cover, δ1 (mm) | δ1=0.9δ≈6.75 |
| Rib | Width, e (mm) | e=0,8÷1δ=6÷7.5 Choose e=7 |
|  | Height, h (mm) | h<58 |
|  | Slope | Approximately 20 |
| Diameters | Based bolt, d1 | d1=0.04a+10=16 >12 |
|  | Bearing side bolt, d2 | d2=(0.7÷0.8)d1=11.2÷12.8=12 |
|  | Body-cover-assembly bolt, d3 | d3=(0.8÷0.9)d2=9.6÷10.8=10 |
|  | Screw for oil caps, d4 | d4=(0.6÷0.7)d3=6÷7=6 |
|  | Screw for top oil cap, d5 | d5=(0.5÷0.6)d3=5÷6=6 |
| The part between the cover and body (mm) | Body’s top thickness, S3 | S3=(1.4÷1.8)d3=14÷18=15 |
|  | Cover’s top thickness, S4 | S4=(0.9÷1)S3=14.4÷16=15 |
|  | Widths (mm) | E2=1.6d2=19 |
|  |  | R2=1.3d2=16 |
|  |  | K2=E2+R2+3÷5=40 |
|  |  | K3=K2-3÷5=38 |
| The bearing seat areas | Bolt center’s diameter of shafts, D2 (mm) | Shaft I: D2=110 |
|  |  | Shaft II: D2=110 |
|  | Outer diameter of shafts, D3 (mm) | Shaft I: D3=135 |
|  |  | Shaft II: D3=135 |
|  | Diameter of screws, d4 | Shaft I and II: d4=M8 |
|  | Height, h (mm) | Shaft I and II: h=12 |
|  | Distance from bolt’s center to edge of hole, k (mm) | k≥1.2d2=14.4 |
| The gearbox body’s bottom (mm) | Thickness, S1 | S1=(1.3÷1.5)d1=20.8÷24 |
|  | Width, K1, q | K1≈3d1=48 |
|  |  | q≥K1+2δ=63 |
| Gap between elements (mm) | Between gears and gearbox inner wall | ∆≥1÷1.2δ=7.5÷9=7.5 |
|  | Between bigger gear and gearbox body’s bottom | ∆1≥3÷5δ=22.5÷37.5=30 |
|  | Between gear faces | ∆≥δ=9 (mm) |
|  | Number of based bolts Z | Z=L+B200÷300=4 |

7.2. Other auxiliary machine elements

    - Ring bolt and hook

To lift and move the gearbox (when manufacturing, assembling…), we usually add ring that C20 steel is chosen as material on its body and lid.

According to reference [1], the center distance of our gearbox is a=150 mm so the weight selected is Q=80 kg, we choose eye bolt M8, C-shaped hook with lifting weight of 80 kg.

Table 7.2 Ring bolt and hook parameter

| Thread d | d1 | d2 | d3 | d4 | d5 | h | h1 | h2 | l | f | b | c | x | r | r1 | r2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M8 | 36 | 20 | 8 | 20 | 13 | 18 | 6 | 5 | ≥18 | 2 | 10 | 1.2 | 2.5 | - | 4 | 4 |

    - Dowel pin

To ensure the relative position of the body and the lid, we use 2 conical dowel pins.

Here, we use the bevel locating bolt because it ensures good positioning due to no gaps, while the cylindrical pin easy to machine but there will be a gap between the pin and the box wall after several times using. According to table 18.4b in the reference [1], the parameters for the pin: d=6 mm; c=1 mm; l=35 mm.

    - Gearbox cap

To seal the bearing, prevent dust, and secure the outer ring of the bearing on the housing. We use 2 types of caps, which are:

Solid gearbox Cap: a cover that does not have a hole or an opening for a shaft to pass through.

Shaft - passing Gearbox Cap: a cover designed with a hole or opening that allows a shaft to pass through.

The bearing cap is molded from gray cast iron GX15-32. From table 18-2 [1]:

Table 7.3 Gearbox cap parameter

| Gearbox cap | D (mm) | D2 (mm) | D3 (mm) | D4 (mm) | h (mm) | Z | d4 (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I | 90 | 110 | 135 | 85 | 12 | 6 | M8 |
| II | 90 | 110 | 135 | 85 | 12 | 6 | M8 |

    - Access door

We make an access door on top of the housing, on which there is an air vent, to check the inside the gearbox and for pouring oil. The parameters based on table 18.5 in reference [1]:

Table 7.4 parameter of access door

| A | B | A1 | B1 | C | C1 | K | R | Screw | Quantity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 150 | 100 | 190 | 140 | 175 | - | 120 | 12 | M8 x 22 | 4 |

    - Oil drain plug

After a period of working, the oil will be contaminated (because of dust and other factors), we need to replace the oil. To drain the contaminated oil, an oil discharge is designed at the bottom of the gearbox.

According to table 18.7 in the reference [2], the parameters for the plug are:

Table 7.6 Oil drain plug parameter

| d | b | m | f | L | c | q | D | S | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M16×1.5 | 12 | 8 | 3 | 23 | 2 | 13.8 | 26 | 17 | 19.6 |

    - Oil dipstick

To check the oil level, we use the level indicator. To avoid interference from the oil wave, the indicator has a cover.

Table 7.7 Oil level indicator parameter

| d | d1 | d2 | D | D1 | L1 | l | l1 | b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M12×1.25 | 5 | 6 | 18 | 12 | 30 | 12 | 6 | 3 |

CHAPTER 8.  THE TOLERANCE

8.1. Bearing’s tolerance

The inner ring of the bearing experiences cyclic loads, so it is chosen to be assembled using an intermediate shaft system to ensure that the inner ring does not slip on the shaft surface during operation. Hence, we must opt for a k6 fit class to ensure even bearing wear during rotation.

The outer ring of the roller bearing does not rotate and thus experiences localized loads. It is assembled using a hole system. To allow the bearing to move axially as the temperature increases during operation, we choose an intermediate fit type of H7.

8.2. Assembly elements

8.2.1 Assembly of the gears and the shafts.

For the gears mounted on the shaft, the fitting type chosen is H7/k6.

8.2.2 Assembly of the bearing cap and the housing body.

To facilitate disassembly and adjustment, we choose a loose fit type of H7/h6.

8.2.3 Assembling of the oil shield on the shaft

To facilitate disassembly, we choose an intermediate fit type of H7/k6.

8.2.4 Assembly of the Positioning Pins

To ensure concentricity and prevent loosening, we choose a fit type of H7/p6.

8.2.5 Assembly of the key

In the axial direction, the fit type on the shaft is P9, and the fit type on the bearing is D10.

8.2.6 Assembly of Positioning Pins

To ensure concentricity and prevent loosening, we choose a fit type of H7/p6.

8.2.7 Assembly of Threads

In the axial direction, the fit type on the shaft is P9, and the fit type on the bearing is D10.

8.3.  Summary of assembly tolerance

Table 7-1: Assembly tolerance

| Machine elements | Assembly | ES (µm) | es (µm) | EI (µm) | ei (µm) | Nmax (µm) | Smax (µm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gear in shaft II | ∅50H7/k6 | +30 | +21 | 0 | +2 | 21 | 28 |
| Grease seal on shaft I | ∅35H7/js6 | +21 | +8 | 0 | -8 | 8 | 29 |
| Grease seal on shaft II | ∅45H7/js6 | +25 | +8 | 0 | -8 | 8 | 33 |
| Spacer on shaft I | ∅25H8/js6 | +33 | +6.5 | 0 | -6.5 | 6.5 | 39.5 |
| Spacer on shaft II | ∅35H8/js6 | +39 | +8 | 0 | -8 | 8 | 47 |
| V-belt drive in shaft I | ∅20k6 | - | +15 | - | +2 | - | - |
| Coupling in shaft II | ∅32k6 | - | +18 | - | +2 | - | - |

Table 7-2: Tolerance for Angular contact ball bearing

| Inner ring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Element | Assembly | ES | es | EI | ei |
| Shaft I | ∅30k6 | - | +15 | - | +2 |
| Shaft II | ∅40k6 | - | +18 | - | +2 |
| Outer ring |  |  |  |  |  |
| Element | Assembly | ES | es | EI | ei |
| Shaft I | ∅72H7 | +30 | - | +0 | - |
| Shaft II | ∅90H7 | +30 | - | +0 | - |

Table 7-3: Tolerance for Thread Assembly

| Thread Cross-Sectional Dimension bxh | Allowance for the Width of the Thread Groove on the Shaft | Allowance for the Width of the Thread Groove on the Bearing | Allowance Limit on the Shaft, t1 | Allowance Limit on the Bearing, t2 |
| --- | --- | --- | --- | --- |
|  | P9 | D10 |  |  |
| 8x7 | -0.015 | +0.098 +0.040 | +0.2 | +0.1 |
| 12x8 | -0.018 | +0,120 +0,050 | +0.2 | +0.2 |
| 14x9 | -0.061 | +0,120 +0,050 | +0.2 | +0.2 |
| 18x11 | -0.061 | +0,120 +0,050 | +0.2 | +0.2 |

CHAPTER 9. DESIGN OF TRANSMISSION SYSTEM

  - Extrusion machine

Extrusion machine uses compression extrusion technology to create plastic pipes, plastic sheets, plastic floors, plastic films, etc. The main function of the machine is to make a molten plastic mixture in a brief time to shape products. After that, the plastic will immediately cool down and maintain its solid shape according to the product extrusion mold.

Extrusion machines play a key role in the plastic production line, specifically as follows:

- As equipment used to shape and produce plastic products, the adjusting machine is the most important in a complete plastic production line.

- Can manage a variety of thermoplastics such as PP, PE, ABS, PVC, PS, PET, ...

- Quality extrusion machines also help the plastic to be shaped quickly, creating beautiful, uniform finished products.

Regardless of the application, a gear driver drives the extrusion machine. In the extrusion machine, there will be a shaft connected to the output of the gearbox. When the motor rotates, this shaft will rotate, and the mixing tank system will also rotate to perform its function.

The transmission systems used in extrusion machines are truly diverse, such as: worm gear drive, coaxial gear drive with spur gears, helical gears, bevel gears, ...

In the scope of this project, the transmission is designated as a 1-stage helical spur gear reducer, and the output shaft is connected to a V – belt drive before reaching the working shaft.

Figure 8-1: Extrusion machine

Figure 8-2: Holton Crest Continuous Rotary Extrusion Machine

  - Spiral screw

Figure 8-3: Single screw extruder

Single screw extrusion is one of the main types of polymer processing operation. The principal function of a single screw extruder is to build up pressure in the polymer melt so that the polymer can be extruded through the die. Most of the single screw extruders are plasticating which mean that the solid resin balls or powders melt in the screw due to the pressure. However, some single screw system can be used for mixing also. Single screw extruder is useful when pure polymer like HDPE is used.

Single screw extruder consists of a screw, barrel, drive mechanism, resin feed arrangement and controls. The constantly turning screw moves the resin through the heated barrel where it is heated to proper temperature and blended into a homogeneous melt. A turbulent back pressure is built up which pushes the melt out of the extruder in the shape of the die. The resin sometimes is not completely melted in the basic extrusion screw. The barrier screw in designed to counter this problem. Sometimes, additional flights are attached to the transition section to separate molten and solid plastic to different channels. As the solid pellet moves forward it melts due to shear against the wall and thus melts and flows into the liquid channel. Thus, the solid channel narrows gradually and the liquid channel widens.

The ability of a screw to manufacture products of superior quality with high productivity and low cost is called its performance. At the design stage of an extrusion Single Screw Extruder process, evaluation of screw performance is especially important. The deformation measure or stretching which materials undergo due to the regular flows inside a conventional screw channel increases linearly with the extruder channel length.

  - Design for spiral screw

The pitch of the screw blade not only determines the rising angle of the screw, but also determines the sliding surface of the material under a certain filling coefficient, so the pitch directly affects the conveying process of the material.

For solid blade, D is the diameter of blade, the pitch of screw blade can be determined:

S=0.8D=168 (mm)#

Figure 8-4: Structural parameters of Screw blade diameter and pitch

# REFERENCES

[1] Trịnh Chất, Lê Văn Uyển (2007), Tính toán thiết kế hệ dẫn động cơ khí, Tập 1&2, NXB Giáo dục, Hà Nội.